Приложение №3

к Положению организации и осуществлению закупок ЗАО «Кумтор Голд Компани»

ТЕХНИЧЕСКОЕ ЗАДАНИЕ

на закуп пожарной автоцистерны (АЦ) для БПБ ЗАО «Кумтор Голд Компани»

| № п/п | Перечень основных данных и требований | Основные данные и требования |
| --- | --- | --- |
| 1. | Место доставки | Иссык-Кульская область, г.Балыкчы, Балыкчинская перевалочная база ЗАО «Кумтор Голд Компани» |
| 2. | Заказчик | ЗАО «Кумтор Голд Компани» |
| 3. | Цель закупки | Обеспечение |
| 4. | Общие требования | АЦ должна быть изготовлена на автомобильном шасси с колесной формулой 4х2, с дизельным двигателем максимальной мощностью не менее 136 кВт (185 л.с.). • Год изготовления АЦ – не ранее 2024 г. • Экологический класс двигателя: не ниже EURO 2. • Основной цвет покрытия наружной поверхности — красный, контрастирующий цвет декоративных полос - белый. • Технические характеристики пожарной автомашины, пожарно-технического вооружения и оборудования должны соответствовать государственным стандартам стран производителей. |
| 5. | Индивидуальные требования | Требования к кабине. Кабина боевого расчета (КБР) четырехдверная модульного типа, однообъемная, должна состоять из двух секций (переднего и заднего салона) и обеспечивать размещение 6 человек боевого расчета, включая водителя. Передний салон – кабина водителя, поставляемая с шасси. Задний салон – должен быть из металлического каркаса, обшитого металлическими листами, с экстерьером из травмобезопастных и ударопрочных материалов, схожих с материалами переднего салона. Силовой каркас заднего салона должен быть сварным, изготовленным из стальных прямоугольных труб. Передняя и задняя стенки заднего салона, крыша, пол должны быть выполнены из стального листа. Аудиовизуальный контакт боевого расчета между передним и задним салонами должен обеспечиваться, через проем в смежных стенках секций. Передний салон КБР должен обеспечивать размещение 2 человек боевого расчета (включая водителя). Задний салон КБР должен обеспечивать размещение 4 человек боевого расчета. Сиденья в задней секции КБР должны быть выполнены в виде рундука, в спинках сидений должны быть оборудованы места для крепления дыхательных аппаратов для пожарных в количестве 4 шт. и 4 шт. запасных баллонов к ним и оснащены ремнями безопасности на всех членов боевого расчета. Подушки сидений КБР должны быть откидными для доступа к рундукам под сиденьями. Ширина рабочего пространства для водителя должна составлять не менее 800 мм. Ширина сидений для каждого сидящего рядом с водителем не менее – 450 мм. Кабина экипажа должна быть оборудована отопителем, обеспечивающим поддержание температуры в салоне в холодный период года не ниже 15оС во всем диапазоне условий эксплуатации. Двери должны открываться по ходу автомобиля и иметь запирающие устройства с наружными и внутренними ручками управления. Внутренние замки должны иметь устройство, исключающее возможность их непроизвольного открытия в движении сидящим в машине боевым расчетом. Ручки запирающих механизмов должны иметь закругленную форму, исключающую причинение травм. Кабина должна быть оборудована травмобезопасными подножками и поручнями. В салоне должно быть предусмотрено место для установки огнетушителя вблизи сиденья водителя. В салоне должно быть предусмотрено место для размещения медицинской транспортной аптечки. В кабине должны иметься таблички и схемы, поясняющие порядок пользования органами управления шасси. Проемы, места ввода органов управления, сигнализации и освещения должны иметь уплотнения, препятствующие проникновению в кабину пыли, грязи, атмосферных осадков и потере тепла. Требование к емкости для воды. На автоцистерне (АЦ) должна быть установлена цистерна для воды, изготовленная из конструкционной стали толщиной не менее 3 мм, толщина нижней части (пола) не менее 4мм, объемом не менее 4000 литров. Цистерна должна быть продольного расположения на эластичных резиновых амортизаторах, установлена без жесткой связи с кабиной и насосным отсеком. Для предохранения от коррозии на внутренние и наружные поверхности цистерны должен быть нанесен грунт и эмалированное покрытие, на нижние, со стороны земли, поверхности цистерны дополнительно должна быть нанесена антикоррозионная шумоизоляционная полимерно-композитная мастика. Для обеспечения осмотра и технического обслуживания цистерна должна иметь люк. Расположение внутренних элементов цистерны не должно препятствовать ревизии их внутренней полости и замене поврежденных элементов. Цистерна должна быть оборудована устройством для непрерывного или дискретного контроля уровня заполнения и расхода воды. Использование стеклянных трубок в указателях уровня не допускается. Указатель уровня должен быть доступен для четкого визуального контроля с рабочего места оператора. Внутри цистерны должны быть расположены перегородки (волноломы), обеспечивающие гашение колебаний жидкости при движении автомобиля. Волноломы должны делить цистерну на сообщающиеся отсеки объемом не более 1500 литров каждый. Цистерна должна иметь устройства, предотвращающие создание в ней избыточного давления при заполнении, разрежения при опорожнении с помощью насоса. Конструкция цистерны должна обеспечивать опорожнение насосом или свободным сливом. Цистерна должна иметь возможность заполняться как собственным насосом, так и сторонним источником. Устройство для слива воды при переполнении цистерны во время заправки должно быть расположено в зоне, исключающей попадание воды на тормозные механизмы колес при заправке и в движении. Требования к емкости для пенообразователя. Бак для пенообразователя должен быть выполнен из материалов, не подверженных коррозии, и иметь объем не менее 240 л. Пенобак должен иметь устройство против вспенивания пенообразователя при заправке, а также устройство, исключающее создание разрежения в баке при подаче пенообразователя. Пенобак должен располагаться в насосном отсеке в задней части пожарной автоцистерны. Все трубопроводы для подачи пенообразователя должны быть изготовлены из коррозионностойкой (нержавеющей) стали. Соединение бака для пенообразователя с насосной установкой должно иметь эластичные элементы, исключающие возникновение в баке усталостных разрушений под воздействием вибрационных нагрузок от шасси и насоса. Требования к кузову. Кузов является составной частью надстройки пожарной автоцистерны и служит для размещения и защиты оборудования от повреждений при транспортировке. Отсеки кузова АЦ должны располагаться на кронштейнах, жестко закрепленных к силовым элементам, закрепленным на раме шасси. Кузовная надстройка должна быть исполнена из сварного каркаса из стальных профилей и обшивки, выполненной из листов металла закреплённых на каркасе с применением клеевых технологий с защитой от щелевой коррозии. По каждому борту кузова должно быть выполнено не менее чем по 2 отсека. Двери отсеков для ПТВ шторного типа. Двери оборудованы запорными устройствами, удерживающими их в открытом и закрытом положениях и сигнализацией открытого положения дверей с индикацией ее в кабине водителя. Проемы дверей, крышки люков и других элементов кузова должны иметь уплотнения, предохраняющие отсеки от попадания в них атмосферных осадков и грязи. Размещение ПТВ в отсеках должно учитывать тактику его оперативного использования, обеспечивать надежность фиксации оборудования, доступность, удобство и безопасность при съеме и установке. Размещение ручных лестниц на кузове пожарного автомобиля должно обеспечивать возможность легкого их съема без помех и съема другого оборудования. Для доступа к оборудованию, расположенному на крыше, в задней части, АЦ снабжена специальной лестницей. Крепление осуществляется на задней стенке кузова таким образом, чтобы не мешать подъему на крышу при любом положении двери заднего отсека. Установка кузова не должна нарушать параметров проходимости. Запасное колесо на период транспортирования должно устанавливаться под лонжеронами шасси в задней части АЦ. Углы въезда и съезда пожарных автомобилей должны быть не менее значений, установленных для базового шасси. Для повышения коррозионной стойкости кузова, обеспечения эстетичного внешнего вида – при изготовлении автоцистерны применяются клеевые технологии крепления обшивки кузова. Требования к насосу. Тип насоса Центробежный Номинальная подача (л/с) не менее 40 Номинальный напор (м) не менее 100 Номинальная частота вращения насоса - фиксированное нормативное значение, при котором подтверждаются номинальные параметры подачи и напора (об/мин) 2700 Наибольшая геометрическая высота всасывания (м) не менее 7,5 Подача насоса при работе от водоема с наибольшей геометрической высотой всасывания (л/с) не менее 35 Вариант исполнения рабочих органов насоса по классу коррозионной стойкости корпус и рабочее колесо насоса должны быть изготовлены из алюминиевых сплавов. Управление трубопроводной арматурой насоса – ручное (маховики и рукоятки). Управление оборотами насоса с помощью тросовой передачи путем изменения оборотов шасси. Насос оборудован полуавтоматической вакуумной системой водозаполнения, системой дозирования пенообразователя, показывающими приборами – манометрами и электронным тахометром, совмещенным со счетчиком времени наработки. Время заполнения насоса с наибольшей геометрической высоты всасывания (с) не более 40. Тип системы пеносмешения эжекционная, обеспечивающий подачу пенообразователя на вход пожарного насоса. Максимальная производительность системы пеносмешения по раствору пенообразователя с концентрацией 6% (л/с) не менее 40. Управление расходом ПО при помощи дозировочной шкалы, обеспечивающей концентрацию раствора 3% и 6%. Требования к запорной и соединительной арматуре. Коммуникации, входящие в состав насосной установки, должны иметь от цистерны развязку (эластичные элементы) и должны быть оборудованы запорной и соединительной арматурой, позволяющей осуществлять функционирование АЦ. Высота размещения напорных патрубков относительно поверхности дороги (площадки оператора) - не более 1300 мм. Конструкция перекрывной арматуры должна исключать возможность возникновения гидроудара. Должна быть предусмотрена возможность полного слива жидкости из всех емкостей и рабочих объемов водопенных коммуникаций и насоса. Располагать сливные пробки следует в местах, обеспечивающих свободный доступ и возможность выпуска жидкостей без попадания на поверхности прилегающих элементов автомобиля. Элементы трубопроводов в системе водопенных коммуникаций могут иметь овальность не более 10 % от номинального диаметра трубы, если иные требования не предусмотрены в рабочей документации. Резкие переходы без радиусов закруглений, наплывы, уступы в коммуникациях не допускаются. В местах неподвижных соединений насоса не допускается течь и каплеобразование жидкости. Требования к системе управления. Все указатели и сигнальные устройства должны быть видны с рабочего места оператора, а показания легко читаться в любое время суток. Маховики и рукоятки органов управления должны изготавливаться или иметь покрытия из малотеплопроводного материала. Рукоятки (рычаги) управления насосной установкой должны иметь форму, обеспечивающую работу в рукавицах. Возле каждого органа управления должна быть маркировка, определяющая его назначение и положение. Маркировка не должна располагаться на съемных частях, если эти части подлежат демонтажу при оперативном использовании АЦ. Требования к дополнительному электрооборудованию. Дополнительное электрооборудование должно обеспечивать: - освещение места работы в темное время суток; - подачу специальных звуковых и световых сигналов; - освещение отсеков; - в кабине боевого расчета сигнализацию об открытых дверях кабины боевого расчета, кузова; - работу воздушных отопителей; - работу средств связи, контрольных приборов и т.д. Освещение отсеков кузова – индивидуальное, при открытии двери отсека включается только освещение открытого отсека. Внутреннее освещение АЦ должно обеспечивать четкую видимость маркировки, делений на шкалах указателей, измерительных и контрольных приборов. Салон должен быть оборудован плафонами внутреннего освещения. АЦ должна быть оснащена выключателем аккумуляторной батареи (выключателем массы) базового шасси. АЦ должна быть оборудована счетчиком времени наработки насоса. Электрическая проводка, проходящая внутри отсеков кузова, должна иметь разъемы, позволяющие демонтировать узлы и агрегаты пожарной надстройки без демонтажа электропроводки. Каждая электрическая цепь питания любого элемента дополнительного электрооборудования должна оснащаться плавким предохранителем (или автоматическим размыкателем), облегчающим быстрый поиск неисправностей в системе. Предохранители рекомендуется монтировать в одном блоке. Все провода должны быть надежно защищены и прочно укреплены, чтобы исключалась возможность их обрыва, перетирания или износа. Отверстия и вырезы для прохода проводов через элементы кузова или шасси должны иметь защиту (обработку или обрамление) для предохранения изоляции проводов от повреждения. Все провода должны быть надежно изолированы и выдерживать воздействие температуры и влажности, которому они подвергаются. Выводы электрооборудования (контактные зажимы и т.п.) и провода должны иметь маркировку. Автомобильный генератор должен обеспечивать соответствующую мощность для бесперебойной работы электропотребителей и других компонентов пожарного автомобиля. Место размещения заземляющего зажима должно быть электрически связано с выходными патрубками для подачи воды от насоса. Дополнительные требования. • АЦ должна иметь заднее защитное устройство. • АЦ должна быть оборудована звуковым сигналом и световым освещением заднего хода. • АЦ может быть укомплектована набором пожарно-технических вооружений, специальным оборудованием и инструментом. • Насосная установка обеспечивает подачу огнетушащих веществ к месту пожара через напорные патрубки. • Предусмотреть забор воды в пожарный насос из цистерны, открытого водоёма или гидранта. Предусмотреть забор пенообразователя из штатного пенобака или из сторонней ёмкости. • Для освещения зоны посадки и высадки боевого расчета, над каждой дверью располагается светодиодный фонарь, включающийся от концевого выключателя, установленного в дверной проем кабины. • Устанавливается сигнально-говорящее устройство (СГУ) с сигнально-проблесковым маяком (синего цвета) на кузове. • Устанавливается защита задних фонарей и маяка. • Устанавливается на задние колеса подкрылки (локера) для защиты от попадания грязи между цистерной и отсеками. |
| 6. | Условия оплаты и сроки поставки | - Условия оплаты рассматриваются в договоре. - Доставка осуществляется производителем до БПБ – в сроки, установленные в договоре. |
| 7. | Гарантийный срок | Гарантийный срок эксплуатации АЦ должен быть не менее 12 месяцев. |
| 8. | Услуги Заказчика | - Внутренняя транспортировка. |
| 9. | Прием, передача и запуск | - Акт приемки будет подписан после полной готовности АЦ в эксплуатацию. - Все затраты за дополнительные работы выявленные в ходе проверки оборудования несет производитель. |
| 10. | Обучение и руководство | - Производитель обязуется провести обучение персонала по обслуживанию и эксплуатации АЦ. - Предоставить руководство по эксплуатации АЦ. - Предоставить парт каталог в электронном и твердом формате. |
| 11. | Критерии оценки качества | - После ввода в эксплуатацию в течение гарантийного срока производится оценка оборудования и должна соответствовать вышеуказанным нашим требованиям. - В случае выявления несоответствий наших требований производитель берет на себя все затраты по их устранению. |
| 12. | Экологические требования | Все работы должны проводиться согласно внутренним регламентам КГК. |
| 13. | Безопасность | - АЦ должна соответствовать требованиям безопасности, установленным действующим законодательством КР. - Все работы должны проводиться согласно внутренним регламентам КГК. |

Appendix #3 to the Regulations on the

Organization and Implementation of

Procurement of Kumtor Gold Company CJSC

TERMS OF REFERENCE

for the purchase of a fire tank truck (Fire Tanker) for the BMY of Kumtor Gold Company CJSC

| # | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1. | Place of delivery | Issyk-Kul region, Balykchy town, Balykchy Marshalling Yard of Kumtor Gold Company CJSC |
| 2. | Customer | Kumtor Gold Company CJSC |
| 3. | Purpose of procurement | Provision |
| 4. | General requirements | The Fire Tanker must be manufactured on a 4x2 vehicle chassis with a diesel engine with a maximum power of at least 136 kW (185 hp). • The manufacture year of the Fire Tanker is no earlier than 2024. • Engine emission class: not lower than EURO 2. • The main color of the outer surface coating is red; the contrasting color of the decorative stripes is white. • The technical characteristics of the Fire Tanker, fire-fighting weapons and equipment must comply with the state standards of the countries of origin. |
| 5. | Individual requirements | Cab requirements. The combat crew cabin (CCC) is a four-door modular type, single-volume, should consist of two sections (front and rear cabin) and provide accommodation for 6 crew members, including the driver. The front cab is the driver’s cab supplied with the chassis. The rear cabin should be made of a metal frame, sheathed with metal sheets, with an exterior made of trauma-proof and impact-resistant materials like the materials of the front interior. The load-bearing frame of the rear cabin should be welded, made of rectangular steel pipes. The front and rear walls of the rear cabin, the roof, and the floor must be made of steel sheets. Audio-visual contact of the combat crew between the front and rear cabins must be provided through an opening in the adjacent walls of the sections. The front cabin of the CCC should provide accommodation for 2 people on the combat crew (including the driver). The rear cabin of the CCC should provide accommodation for 4 people on the combat crew. The seats in the rear section of the CCC should be made in the form of a locker, in the backs of the seats there should be places for attaching breathing apparatus for firefighters in the amount of 4 pcs. and 4 pcs. of the spare cylinders for them and equipped with seat belts for all members of the combat crew. CCC seat cushions should be folding to access the lockers under the seats. The width of the working space for the driver should be at least 800 mm. The width of the seats for each person sitting next to the driver should be at least 450 mm. The crew cabin shall be equipped with a heater capable of maintaining an interior temperature of no less than 15°C during cold seasons across the full range of operating conditions. The doors should open in the direction of vehicle movement and be fitted with locking mechanisms operable from both the inside and outside. Interior locks should include a safety feature that prevents accidental opening by crew members while the vehicle is in motion. Door handle mechanisms should have rounded shapes to prevent injury. The cabin must be equipped with injury-safe footrests and handrails. A place must be provided in the passenger compartment for the installation of a fire extinguisher near the driver's seat. The cabin must have a place for placing a medical transport kit. The cab must have plates and diagrams explaining the procedure for using the landing gear controls. Openings, entry points of controls, alarms and lighting must have seals that prevent the penetration of dust, dirt, precipitation into the cabin and heat loss. Requirement for the water tank. A water tank made of structural steel with a thickness of at least 3 mm, a thickness of the lower part (floor) of at least 4 mm, a volume of at least 4000 liters must be installed on a tank truck (Fire Tanker). The tank should be longitudinally located on elastic rubber shock absorbers, installed without a rigid connection with the cabin and pump compartment. To protect against corrosion, primer and enamel coating should be applied to the inner and outer surfaces of the tank, and an anti-corrosion soundproofing polymer-composite mastic should be additionally applied to the lower surfaces of the tank from the ground side. To ensure inspection and maintenance, the tank must have a hatch. The location of the internal elements of the tank shall not prevent the inspection of their internal cavity and the replacement of damaged elements. The tank shall be equipped with a device for continuous or discrete monitoring of the filling level and water flow. The use of glass tubes in level indicators is not permitted. The level indicator must be accessible for clear visual control from the operator's station. Inside the tank, there should be partitions (breakwaters) that provide damping of fluid vibrations when the vehicle is moving. Breakwaters must divide the tank into communicating compartments with a volume of no more than 1500 liters each. The tank shall be equipped with devices to prevent the creation of overpressure when filling, vacuum during emptying by means of a pump. The tank shall be designed to be discharged by pump or free discharge. The tank must be filled both by its own pump and by a third-party source. The device for draining water when the tank is overfilled during refueling should be in an area that prevents water from getting on the brakes of the wheels during refueling and in motion. Requirements for the foam concentrate container. The foam concentrate tank must be made of non-corrosive materials and have a volume of at least 240 liters. The foam tank shall be in the pump compartment at the rear of the fire tanker. All pipelines for the supply of foam concentrate must be made of corrosion-resistant (stainless) steel. The connection of the foam concentration tank with the pumping unit must have elastic elements that exclude the occurrence of fatigue failures in the tank under the influence of vibration loads from the chassis and pump. Requirements for the body. The body is an integral part of the superstructure of a fire tank truck and serves to accommodate and protect equipment from damage during transportation. The compartments of the Fire Tanker body must be located on brackets rigidly fixed to the load-bearing elements fixed to the chassis frame. The body superstructure should be made of a welded frame of steel profiles and sheathing made of metal sheets fixed to the frame using adhesive technologies with protection against crevice corrosion. At least 2 compartments must be made on each side of the body. The compartment doors for fire and rescue equipment (FRE) are of curtain type. The doors are equipped with locking devices that keep them in the open and closed positions and an alarm of the open position of the doors with its indication in the driver's cab. Door openings, hatch covers, and other body elements must have seals that protect the compartments from precipitation and dirt. Placement of FRE in compartments should consider the tactics of its operational use, ensure the reliability of equipment fixation, accessibility, convenience and safety during removal and installation. The placement of hand ladders on the body of a fire truck should provide the possibility of their easy removal without interference and removal of other equipment. To access the equipment located on the roof, in the rear part, the Fire Tanker is equipped with a special ladder. Fastening is carried out on the rear wall of the body in such a way as not to interfere with the rise to the roof at any position of the rear compartment door. The installation of the body should not violate the cross-country parameters. The spare wheel for the period of transportation must be installed under the chassis spars in the rear part of the Fire Tanker. The angles of entry and departure of fire trucks must be not less than the values established for the base chassis. To increase the corrosion resistance of the body, to ensure an aesthetic appearance, adhesive technologies for fastening the body trim are used in the manufacture of the tanker. Requirements for the pump. Pump Type: Centrifugal Rated flow (l/s) not less than 40. Rated head (m) not less than 100. Nominal speed of the pump is a fixed standard value at which the nominal parameters of the flow and head (rpm) are confirmed - 2700. Maximum geometric suction height (m) not less than 7.5 Pump supply when operating from a reservoir with the highest geometric suction height (l/s) of at least 35. Version of the pump working bodies according to the class of corrosion resistance, the pump casing and impeller should be made of aluminum alloys. Pump valves are controlled manually (handwheels and handles). Controlling the speed of the pump using a cable transmission by changing the speed of the chassis. The pump is equipped with a semi-automatic vacuum water filling system, a foam concentration dosing system, indicating devices - pressure gauges and an electronic tachometer combined with an operating time counter. The time of filling the pump from the maximum geometric suction height (s) is not more than 40. The type of foam mixing system is ejection, which ensures the supply of foam concentrate to the inlet of the fire pump. The maximum capacity of the foam mixing system for a foam concentrate solution with a concentration of 6% (l/s) of at least 40. The foam agent (FA) flow shall be controlled using a dosing scale that ensures solution concentrations of 3% and 6%. Requirements for shutting off and connecting valves. Communications included in the pumping unit must have an interchange (elastic elements) from the tank and must be equipped with shut off and connecting valves that allow the operation of the Fire Tanker. The height of the discharge pipes relative to the road surface (operator's platform) is not more than 1300 mm. The design of the shut-off valve must exclude the possibility of water hammer. It should be possible to completely drain the liquid from all tanks and working volumes of water-foam communications and the pump. Drain plugs should be in places that provide free access and the ability to release liquids without getting on the surface of adjacent elements of the car. Pipeline elements in the water-foam communications system may have an ovality of no more than 10% of the nominal diameter of the pipe unless other requirements are provided for in the working documentation. Sharp transitions without radii of rounding, sags, ledges in communications are not allowed. In the places of fixed connections of the pump, leakage and dripping of liquid are not allowed. Control System Requirements. All indicators and warning devices must be clearly visible from the operator’s workstation, and their readings must be easily legible at any time of day. Control wheels and handles must be made of, or coated with, low-thermal conductivity materials. Handles (levers) for operating the pump system must be shaped to allow use while wearing gloves. Each control element must be labeled to indicate its function and position. Labels must not be placed on removable parts if those parts are subject to removal during operational use of the Fire Tanker truck. Requirements for additional electrical equipment. Additional electrical equipment must provide: - lighting of the place of work in the dark. - giving special sound and light signals. - compartment lighting. - in the cabin of the combat crew, alarm about the open doors of the cabin of the combat crew, body. - operation of air heaters. - operation of communication facilities, control devices, etc. The lighting of the body compartments is individual; when the compartment door is opened, only the lighting of the open compartment is turned on. The interior lighting of the Fire Tanker should provide clear visibility of markings, divisions on the scales of indicators, measuring and control devices. The cabin should be equipped with interior lighting shades. The Fire Tanker must be equipped with a battery switch (ground switch) for the base chassis. The Fire Tanker must be equipped with a pump operating time meter. Electrical wiring passing inside the body compartments must have connectors that allow dismantling the components and assemblies of the fire superstructure without dismantling the electrical wiring. Each electrical circuit of any element of additional electrical equipment must be equipped with a fuse (or automatic breaker) that facilitates quick troubleshooting in the system. It is recommended to mount fuses in one unit. All wires must be reliably protected and firmly strengthened to exclude the possibility of their breakage, fraying or wear. Openings and cutouts for the passage of wires through body elements or chassis must be protected (treated or framed) to protect the insulation of wires from damage. All wires must be reliably insulated and can withstand the temperature and humidity to which they are exposed. Electrical equipment terminals (contact clamps, etc.) and wires must be marked. A car alternator must provide the appropriate power for the uninterrupted operation of electrical consumers and other components of a fire truck. The location of the grounding clamp should be electrically connected to the outlet pipes for supplying water from the pump. Additional requirements. • The Fire Tanker must have a rear protective device. • The Fire Tanker must be equipped with a sound signal and reversing light. • The Fire Tanker can be equipped with a set of fire-fighting equipment, special equipment, and tools. • The pumping unit provides the supply of fire extinguishing agents to the fire site through pressure pipes. • Provide for the intake of water into the fire pump from a tank, open reservoir, or hydrant. Provide for the intake of the foam concentrate from a standard foam tank or from a third-party container. • To illuminate the embarkation and disembarkation area of the combat crew, an LED light is located above each door, which turns on from a limit switch installed in the doorway of the cabin. • A signal-talking device (STD) with a signal-flashing beacon (blue) is installed on the body. • Rear light and beacon protection is installed. • It is installed on the rear wheels of the fender liner (locker) to protect against dirt ingress between the tank and compartments. |
| 6. | Payment terms and delivery times | - The terms of payment are discussed in the contract. - Delivery is conducted by the manufacturer to the BMY - within the terms established in the contract. |
| 7. | Warranty period | The warranty period of the Fire Tanker must be at least 12 months. |
| 8. | Customer Services | - Internal transportation. |
| 9. | Acceptance, Handover, and Commissioning | - The acceptance certificate will be signed after the Fire Tanker is fully ready for operation. - All costs for additional work identified during the inspection of the equipment are borne by the Manufacturer. |
| 10. | Training and management | - The Manufacturer undertakes to train personnel for the maintenance and operation of the Fire Tanker. - Provide a manual for the operation of the Fire Tanker. - Provide a part catalog in electronic and hard format. |
| 11. | Quality Assessment Criteria | - After commissioning, the equipment is evaluated during the warranty period and must meet our above requirements. - In case of non-compliance with our requirements, the Manufacturer assumes all costs for their elimination. |
| 12. | Environmental requirements | All work must be conducted in accordance with the internal regulations of KGC. |
| 13. | Security | - The Fire Tanker must comply with the safety requirements established by the current legislation of the Kyrgyz Republic. - All work must be conducted in accordance with the internal regulations of KGC. |