Appendix #3 to the Procurement Organization and Implementation Policy of Kumtor Gold Company CJSC

APPROVED BY:

KGC Deputy Chairman of the Board

Head of Structural Unit

(in accordance with the Regulation on the Delegation of Financial Authority)

Name_______________________

“____” ______________2026

Terms of Reference

for the supply of a methyl isobutylcarbinol (MIBC) dosing system for flotation.

| Item # | Requirement parameters for goods, works, services to be purchased (subject of the purchase) | Specific requirements for goods, work, and services |
| --- | --- | --- |
| 1 | Name/type/quantity/ volumes/measure unit | One (1) (1) methyl isobutylcarbinol (MIBC) flotation dosing system |
| 2 | Place of delivery of goods, provision of services/performance of work | Kyrgyz Republic, Issyk-Kul region, Djety-Oguz district, “Kumtor” gold mine |
| 3 | Terms (periods) of delivery of goods, provision of services, performance of work | Determined by the terms of the contract (the delivery schedule is agreed by the parties). |
| 4 | The procedure (sequence, stages) for performing the work | an approximate work schedule, including a breakdown by stages and phases. |
| 5 | Requirements for Supplied Goods, Performed Works, and Provided Services | Requirements for the supplied goods - The Goods shall be new and unused. - Compliance with the functional, technical, and quality characteristics specified in the Terms of Reference. - Application area: operation at high altitude (over 4000 m above sea level). - Characteristics of the substance MIBC (for material selection and safety considerations): - Chemical Name: Methyl Isobutyl Carbinol (MIBC, 4‑Methyl‑2‑Pentanol. - Application: frother for flotation processes. - Operating mode: continuous/periodic dosing into tanks or flotation cells. - Operating temperature: 5... 40 °C. - Specific gravity: 0.806... 0.809 g/cm³ at 20 °C. - Flash point: ~40 °C (open cup). - Hazard class: flammable liquid, Category 3 (H226). - Toxicity: causes eye and skin irritation (H319, H335). - Storage: in a cool, dry, well-ventilated area, away from ignition sources. 5. Design scope The system must be skid-mounted and include storage tank, pumps, piping, calibration column, control system, and all safety elements. Explosion protection: suitable for Zone 2, Group IIB, Temperature Class T3 (or equivalent), compliant with ATEX/IECEx. 6. Typical system composition - Tank: SS316L or antistatic HDPE; bunding for 110% of volume; flame arrester; level sensor; vent filter. - Dosing pumps: 2×100% (operating/standby), diaphragm or gear; adjustable flow 0.5-20 l/h; ATEX-certified motors. - Suction line: shut-off valve; check valve with strainer; calibration column ~1 l. - Discharge line: pulsation dampener; safety valve; pressure gauge; check valve; flow meter; injection quill. - Control panel: PLC with local/remote modes; flow-proportional dosing; safety interlocks. - Safety features: grounding; bunding; fire extinguishers; emergency shower/eye bath; ventilation. 7. Additional data - Number of dosing points: 5 pump sets (per point: working + backup). - Pump discharge pressure: 3 sets rated 0-5 bar; 2 sets rated 0-10 bar. - Dosing range per point: 10 mL – 300 mL. - Tank volume: ~2000 l. - Construction material: stainless steel. - Electrical classification: Ex, category 3. - Power supply: 220 V AC. Signals: 24 V DC (DCS/analog). Flushing medium: water. 8. Construction materials Wetted parts: SS316L, PTFE/PFA, FFKM or FKM. Skid frame: Painted carbon steel or SS304. Tubing/Fittings: SS316L or PTFEliner. Gaskets: PTFE. Electrical equipment: explosion-proof, ATEX certified. 9. Control system and instrumentation - PLC-based control with local HMI (manual/auto), the ability to connect to the process control system via Ethernet. - Level transmitter in storage tank (4-20 mA) with alarm outputs. - Discharge pressure transmitter (4-20 mA) and safety relief valve. - Flow meter with totalizer (4-20 mA). Interlocks: low level, high pressure, and access to the panel. Equivalent/superior solutions are acceptable with full technical justification. |
| 6 | Procedure for Delivery and Acceptance of Goods, Services, Work Results | - If any non‑conformities with the requirements are identified, the Supplier shall correct them at its own expense. - All costs associated with additional work identified during the inspection of the equipment shall be borne by the Supplier. |
| 7 | Requirements for the submission of technical and related documentation to the Customer upon completion and delivery of the work | Upon delivery, the Supplier shall provide: - equipment passports/certificates. - documents confirming the Manufacturer's/Supplier's warranty. - GA and P&ID drawings, electrical schematics, and control logic descriptions. - ATEX/IECEx certificates. - FAT program and FAT report. - list of recommended spare parts (for 2 years). - the User manual and technical documentation in a complete set. |
| 8 | Warranty obligations | The Supplier shall provide a warranty for the supplied equipment for a period of not less than 12 months from the date of commissioning. The warranty obligations include:- free elimination of all defects arising due to the fault of the manufacturer or the supplier;- replacement of faulty components and parts at the Supplier’s expense;- consultation and technical support throughout the warranty period;- the Supplier’s obligation to ensure the availability of spare parts;- dispatch of the Supplier’s specialist if necessary (if stipulated in the contract). The warranty does not cover damage caused by violation of the operating, storage, or installation requirements. |

Prepared by:

Position                                        					Name

Position								Name