Terms of Reference

for the purchase of cylindrical shell segments and a flange

| Item № | Requirement parameters for goods, works, services to be purchased (subject of the purchase) | Specific requirements for goods, work, and services |
| --- | --- | --- |
| 1 | Name/type/quantity/volumes/measure unit | - Cylindrical shell segment № 1: 1 pc. Manufactured in the form of two separate segments (two half-shells). - Outer diameter: 1,849 mm. - Length: 740 mm. - Wall thickness: 10 mm. - Material: 253MA. - Cylindrical shell segment № 2: 1 pc. Manufactured in the form of two separate segments (two half-shells). - Outer diameter: 1,849 mm. - Length: 400 mm. - Wall thickness: 10 mm. - Material: 253MA. - Cylindrical shell segment № 3: 1 pc. Manufactured in the form of two separate segments (two half-shells). - Outer diameter: 1,808 mm. - Length: 1,600 mm. - Wall thickness: 10 mm. - Material: 316L. 4. Mating flange for the end seal (hereinafter referred to as “Flange”): 2 pcs. - Manufactured of two parts: - Outer part in the form of crescents made of 50×50 angle bars. - Inner part manufactured of plates according to the drawing. - Assembled with M10 bolts. - Dimensions are shown in the attached drawing. - Thickness of plates: 6 mm. - Flange material: 316L. |
| 2 | Delivery location | 9 Naryn highway, Balykchy, Issyk-Kul region, Kyrgyz Republic. |
| 3 | Delivery Deadline | - Delivery deadlines shall be specified in the Commercial Proposal. - The Client shall select proposals with optimal prices and delivery terms that meet the requirements of standards and the Terms of Reference for product quality. - Delivery terms shall be agreed upon and specified in the Agreement. |
| 4 | Procedure (sequence, stages) of works | - Submission of a Commercial Proposal specifying the following: - Product price. - Certificates of conformity of the material. - Certificates of conformity for the work performed in accordance with ASTM A409 standards or equivalent standards. - Manufacturing period. - Delivery terms and conditions. - Preparation of the agreement. - Manufacture of shells. - Manufacture of flange. - Quality control: - Geometry inspection. - Submission of the quality inspection report. - Packaging of pipes and plates. - Shipment. - The products will be accepted at the mine site upon their delivery, with inspection of geometry and weld quality. |
| 5 | Requirements for goods, services and works to be delivered | 1. The manufacturing process shall comply with EN ISO 13920, EN ISO 2768, EN ISO 9692-1, EN ISO 1127, and ASTM A999 standards. 2. The material of the supplied products shall comply with that specified in the product description. 3. Shell segments shall have bevels for welds according to EN ISO 9692-1, type BW-V, with the following parameters: - Opening angle α = 60-70°. - Root face c = 1.5–2.0 mm. 4. The geometry of the pipes shall comply with the product description and EN ISO 1127, ASTM A999 standards, or other applicable international standards. 5. The shells shall be supplied unwelded. The shells shall be welded by the Client on site. 6. The packaging of shells shall protect against mechanical damage, loss of geometry, and prevent contact with carbon steel and moisture. 7. Recommendations for welding 253MA and T316L materials shall be provided. |
| 6 | Work Completion and Acceptance Procedure | - Quality certificates shall be included with the Commercial Proposal. - Quality inspection report for the finished products shall be submitted after manufacturing. - Upon arrival at the mine site, the products shall be inspected for quality by KGC personnel. |
| 7 | Requirements for the submission of technical and related documentation to the Client upon completion and delivery of the work | - Certificates for materials - EN 10204 3.1 or another standard. - Visual inspection report. - Measurement report (ID, OD, ovality, thickness). |
| 8 | Warranty obligations | - During dimensional and geometric inspection, the shells and flanges shall comply with the relevant standards. - In case of non-compliance, the Supplier shall collect the product at its own expense and refund the Client. |