Terms of Reference

for the purchase of 253MA Pipes and T316L Plates

| Item № | Requirement parameters for goods, works, services to be purchased (subject of the purchase) | Specific requirements for goods, work, and services |
| --- | --- | --- |
| 1 | Name/type/quantity/volumes/measure unit | - Pipe with an outer diameter of 1,828.8 mm (6`), length 6,000 mm, wall thickness 10 mm, 253MA material - 2 pcs. - Pipe with an outer diameter of 1,828.8 mm (6`), length 6,288 mm, wall thickness 10 mm, 253MA material – 1 pc. - Plate, 6 mm*50 mm*150 mm, T316L material – 1,500 pcs. - Plate, 6 mm*100 mm*150 mm, T316L material – 2,000 pcs. |
| 2 | Delivery location | - The work shall be conducted at the Supplier’s industrial site. - Delivery: 9 Naryn highway, Balykchy, Kyrgyz Republic. |
| 3 | Delivery Deadline | - Delivery deadlines shall be specified in the Commercial Proposal. - The Client shall select proposals with optimal prices and delivery terms that meet the requirements of standards and the Terms of Reference for product quality. - Delivery terms shall be agreed upon and specified in the Agreement. |
| 4 | Procedure (sequence, stages) of works | - Submission of a Commercial Proposal specifying the following: - Product price. - Certificates of conformity of the material. - Certificates of conformity for the work performed in accordance with ASTM A409 standards or equivalent standards. - Manufacturing period. - Delivery terms and conditions. - Preparation of the agreement. - Production of pipes and plates. - Quality control: - Weld inspection (non-destructive testing, NDT). - Geometry check. - Submission of the quality inspection report. - Packaging of pipes and plates. - Shipment. - The products will be accepted at the mine site upon their delivery, with inspection of geometry and weld quality. |
| 5 | Requirements for goods, services and works to be delivered | 1. The manufacturing process shall comply with ASTM A409, DIN EN 10217–7, ASTM A380, and ASTM A967 standards. (Mandatory heat treatment of welded seams). 2. The material of the supplied products shall comply with that specified in the product description. 3. The pipes shall have beveled ends (chamfers) for welded seams according to the standard. 4. The geometry of the pipes shall comply with the product description and EN ISO 1127, ASTM A999 standards, or other applicable international standards. 5. Non-destructive testing shall comply with EN ISO 17635 or equivalent international standards. 6. The packaging of pipes and plates shall protect against mechanical damage, loss of geometry, and prevent contact with carbon steel and moisture. 7. Recommendations for welding 253MA and T316L materials shall be provided. |
| 6 | Work Completion and Acceptance Procedure | - Quality certificates must be included with the Commercial Proposal. - Quality inspection report for the finished products shall be submitted after manufacturing. - Upon arrival at the mine site, the products shall be inspected for quality by KGC personnel. |
| 7 | Requirements for the submission of technical and related documentation to the Client upon completion and delivery of the work | - Certificates for materials - EN 10204 3.1 or another standard. - NDT reports (VT/PT/RT/UT) according to EN ISO 17635. - Heat treatment report (solution annealing). |
| 8 | Warranty obligations | - When inspecting dimensions and weld quality, the geometry of plates and pipes shall comply with standards. - In case of non-compliance, the Supplier shall collect the product at its own expense and refund the Client. |