TERMS OF REFERENCE

for the supply of a courier bus

| Item № | Section | Specifications and technical requirements |
| --- | --- | --- |
| 1. | Location | 24/1 Ibraimov St., Bishkek, Kyrgyz Republic. |
| 2. | Client | Transportation Services. |
| 3. | Purpose of the purchase | The subject of these Terms of Reference (TOR) is the purchase of 1 (one) unit of tourist bus. |
| 4. | Manufacturing date requirements | The bus shall be brand new, manufactured no earlier than 2025. |
| 5. | Technical specifications | Engine: turbocharged diesel. Emission standard: not lower than Euro 5. Engine power: not less than 355 hp. Transmission: manual, 6-speed, with a transmission retarder. Suspension: pneumatic with body position controls, ECAS (electronically controlled pneumatic suspension system). Steering: height and tilt adjustment. Braking and safety systems: ABS (anti-lock braking system), ESC/ESP/ESR (stability control system), ASR (anti-slip system). Vehicle layout: coach-type with a high luggage compartment. Body: body frame made of high-strength materials. The front-end structural frame is corrosion-protected. A fully load-bearing structure that shall comply with international safety standards. The passenger compartment floor covering shall be made of a modern, high-strength, anti-slip, and moisture-resistant material. Number of passenger doors: 2. Number of passenger seats: at least 50+1. Driver’s seat: air-suspended, equipped with 3-point seat belts, an integrated headrest, and adjustable lumbar support. Passenger seats: equipped with 3-point seat belts, armrests, numbering, aisle-side sliding, adjustable seatback, and USB sockets (optional). Fuel tank: at least 400 liters, made of anti-corrosion material. |
| 6. | Bus equipment, fittings, and accessories. | Bus equipment: - the driver’s cabin and the passenger compartment shall be equipped with emergency hand hammers. - mandatory fire extinguishers, both in the driver's cab and in the passenger compartment. - automatic fire extinguishing system in the engine and autonomous heater compartments. - jack. - spare tire. - a set of standard tools and accessories. - a set of spare parts for the 1st maintenance. - mandatory availability of emergency exits and hatches for emergency evacuation of passengers. - 360 0-degree round view system. - passenger compartment surveillance camera. - radio receiver. - microphone. - all information shall be displayed on the color display installed in the driver's cab. - the bus interior shall be equipped with LED lighting. - handrails and grad handles shall be installed for the safety of passengers. - all information plates and inscriptions placed on the body and inside the bus shall be in Kyrgyz, Russian, and English (all inscriptions and information plates shall be preliminarily agreed by the Supplier with the Purchaser). - fog lights. - parking lights. - heated exterior side mirrors. - rear-view mirror in the driver's cab. - open-type overhead luggage racks above the seats. - windows with tinted glazing. - all windows shall be equipped with factory-installed curtains. - aisle lighting. - separate climate control for the passenger area and the driver's workplace. - shall be equipped with at least two emergency ventilation hatches. - automatic fire extinguishing system in the engine and autonomous heater compartments. |
| 7. | Operating requirements | The bus shall be equipped with a winter package to ensure normal operation in climatic conditions with ambient temperatures ranging from -40°C to +45°C and a relative humidity of 85%. |
| 8. | Standards and certificates | The bus shall comply with international standards. Certificate of Conformity is mandatory. |
| 9. | Vehicle acceptance procedure and personnel training | The bus shall be accepted through the signing by the Parties of a Commissioning Certificate. Training of the Purchaser’s personnel on operation and maintenance shall be conducted during commissioning and start-up works. |
| 10. | Vehicle warranty period | The vehicle warranty period shall be no less than 24 months from the date of commissioning or 100,000 km of mileage, whichever occurs first. |
| 11. | Warranty and technical maintenance | Warranty and post-warranty maintenance and the supply of spare parts shall be provided by the equipment supplier. The availability of a service center for maintenance in Bishkek is mandatory. |
| 12. | Documentation requirement | Сomplete set of documentation required for registering the vehicle with the Center for Vehicle and Driver Personnel Registration under the Administration of the President of the Kyrgyz Republic. Copies of cargo customs declarations. Warranty and service book, operation manual in Russian in both hard copy and electronic format. Spare parts catalog. |