TERMS OF REFERENCE

for the purchase of an off-road container truck for the “Kumtor Gold Company” CJSC

| Item # | List of main data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1. | Place of delivery | Kyrgyz Republic, Balykchy town, Naryn Highway Street, 9. |
| 2. | Customer | Kumtor Gold Company CJSC |
| 3. | Subject of purchase | The subject of the Terms of Reference is the purchase of 1 (one) unit of an off-road container truck. |
| 4. | Purpose of purchase | Renewal of the vehicle fleet of the Crane Service of the Site Services &Special Projects department |
| 5. | General requirements | - The supplied vehicle (hereinafter — the Vehicle) must be new and not previously used. Type: high‑mobility (off‑road capable) cargo truck belonging to category C vehicles. Purpose: movement on public roads for the transportation of goods. Operating conditions: highway and unpaved roads at temperatures from –37°C to +40°C and at altitudes up to 4,000 meters above sea level. - The Vehicle must meet or exceed the performance and ergonomic requirements specified in the technical specifications. - All necessary user manuals and technical documentation must be supplied on the set and shall not be provided as copies. - The Supplier must provide an original spare parts catalog for the supplied Vehicle. - The Vehicle must comply with the quality requirements established by technical regulations, standardization documents, state standards, type approval documents (OTTS), applicable to goods of this type and valid within the territory of the Kyrgyz Republic and the EAEU. - When delivering the Vehicle, the Supplier must provide originals or duly certified copies of valid certificates of conformity and declarations of conformity with regulatory requirements for the supplied goods. - The supplied goods must not be pledged, under arrest, or subject to any other encumbrance. |
| 6. | Technical specifications | The required technical characteristics are provided in Appendix #1 |
| 7. | Payment terms and delivery periods | - To be determined under the terms of the contract in accordance with the Supplier’s commercial offer. - Delivery shall be carried out by the Supplier to the Customer’s designated checkpoint (Balykchy Marshalling Yard - BMY) within the deadlines established by the Customer. - In case of failure to meet the delivery deadlines, penalty sanctions will be considered. - The delivery period for the required equipment shall not exceed 4 months from the date of signing the contract. |
| 8. | Warranty period | - The supplier must provide a warranty for the supplied equipment for at least 100,000 km or 12 months from the date of commissioning / the Vehicle is put into operation. |
| 9. | Quality Evaluation Criteria | - After commissioning and throughout the warranty period, the Vehicle and equipment shall be evaluated. The characteristics of the Vehicle must comply with the Manufacturer’s specifications. - If any non‑compliances with the stated requirements are identified, the Supplier shall bear all costs for their elimination. |
| 10. | Documentation requirements | - Availability of the full set of documents required for vehicle registration with the State Center for Registration of Vehicles and Driver Personnel under the Administration of the President of the Kyrgyz Republic. Copies of cargo customs declarations. Availability of the warranty and service book, and the operation manual in Russian, provided in both paper and electronic formats. |
| 11. | Confidentiality | - Information regarding the transaction shall be considered confidential, and the Supplier shall be liable for its disclosure to third parties. |
| 12. | Safety | - The Vehicle must comply with the safety requirements established by the current legislation of the Kyrgyz Republic. |

Appendix #1 to the Terms of Reference

| 1. Required technical specifications of the chassis. |  |
| --- | --- |
| Vehicle type | Container truck, platform for transporting 20-foot containers. |
| Wheel arrangement | 6 x 6 |
| Payload capacity, t | not less than 10 tons |
| Engine | Diesel turbocharged |
| Engine displacement volume, cm3 | not less than 11000 cm3 |
| Rated power, hp | 300 hp or higher |
| Emission standard | Euro 3-4 |
| Fuel type | Diesel |
| Gearbox/transmission type | Mechanical/manual |
| Axle differential lock | Yes |
| Left-hand steering | Yes |
| Motor brake - retarder | Yes |
| Service Brake System | Pneumatic, with drum-type brakes |
| ABS Anti-lock Braking System | Yes |
| Front suspension | Leaf-spring |
| Rear suspension | Leaf‑spring, balance‑type |
| Fuel tank capacity, l | 210 and more |
| Wheel configuration / tires | three-axle - single‑wheel configuration |
| Tire size | 425/85R21 |
| Front and rear fenders, mudguards, protective covers on axles | Yes |
| Cabin | 3-seater |
| Independent heater (cabin and engine pre‑heater) | Yes |
| On‑board electrical system voltage | 24 V |
| Battery set | 2pcs 190Ah |
| Headlights (low/high beam) and fog lights | Yes |
| Reversing lights and rear fog lamp | Yes |
| Front and rear position lights | Yes |
| Rotating beacon on cabin (yellow) | Yes |
| Convoy/road‑train lights on cabin roof with switch | Yes |
| Horn (pneumatic/electric) | Yes |
| Automatic reverse warning alarm | Yes |
| Spare wheel | Yes |
| Availability of fire extinguishers | Yes |
| 2. Required overall dimensions and platform specifications |  |
| Platform length × width, mm | from 6000x2450 |
| Container type with deck | 20 feet |
| Container platform with decking | Yes |
| Platform deck material Metal, checkered/anti‑slip | Yes |
| Container locking fittings | 4 pcs. |
| Towing device (tow hitch) | Yes |