TERMS OF REFERENCE

for the purchase of “universal sweeping and cleaning machine”

| Item # | Requirement parameters for goods, works, services to be purchased (subject of the purchase) | Specific requirements for goods, work, services |
| --- | --- | --- |
| 1. | Name/type, quantity/volumes, measure unit | Universal sweeping and cleaning machine for year‑round territory maintenance, equipped with a diesel engine (hereinafter – the Machine). Quantity: 1 (one) unit. Measure unit: one piece (EA.). |
| 2. | Place of delivery of goods, provision of services/performance of work | “Kumtor” Balykchy Marshalling Yard, Balykchy town, Naryn highway, 9. |
| 3. | Delivery period for goods, provision of services, performance of works | To be stipulated in the contract. Delivery shall be carried out in a single shipment, with full completeness of the equipment. |
| 4. | Procedure (sequence and stages) for work performance | - Delivery of the Machine to the delivery location. - If necessary – assembly/installation of units and components, preparation for operation. - Commissioning works: verification of the operability of all systems at the Customer’s site (including the sweeping and suction system, brush control, hydraulics, lifting and tipping of the waste container). - Commissioning works: verification of the operability of all systems at the Customer’s site (including the sweeping and suction system, brush control, hydraulics, lifting and tipping of the waste container). - Transfer of operational and other documentation, signing of the acceptance certificate |
| 5. | Requirements for the supplied goods, performed works, and provided services | General requirements: - The Machine must be new, previously unused, not refurbished, and of serial industrial production. - The Machine, in the supplied configuration, must allow immediate commissioning for its intended use without the need to purchase additional equipment (except consumables). - Compliance with the applicable technical regulations, standards, and safety requirements in force in the Kyrgyz Republic regarding machinery, mechanisms, occupational safety, and environmental protection. - The design of the Machine must ensure year‑round operation in the climatic conditions of the Customer’s region (summer and winter periods). Main technical characteristics and functional requirements: - Engine type - diesel. The engine power must ensure the Machine’s performance as specified below without overload. - Presence of a hydraulic system controlling the key Machine components: steering system, brush drives, suction fan drive, lifting and tipping of the waste container. - Suction system with dual air circulation, providing enhanced suction power and efficient debris collection. - Sweeping width with brush set: not less than 2.4 m. - Cleaning capacity is at least 24000 m² / hour. - Waste container volume: not less than 1500 L. The container must be made of corrosion‑resistant material or have anti‑corrosion coating, with hydraulic lifting and tipping for waste unloading. - At least two sweeping brushes with independent adjustment of position and pressure. Brushes must automatically bypass obstacles (curbs, manholes, etc.) without damaging the brushes or the road surface. - Brush suspension without ballast, reducing brush wear and minimizing surface damage during cleaning. - The chassis design and running gear must provide good maneuverability and stability during movement and cleaning on paved surfaces and on slopes permitted by the Manufacturer. - Use of wear‑resistant materials in key units (brush mechanism, suction duct, bottom plate, etc.) to ensure extended service life and reduced operating costs. Operator Cabin and Ergonomics: - A closed, spacious operator cabin with glazing providing good visibility of the working area in the front, sides, and rear. - Equipped with air conditioner and cabin heater, ensuring comfortable operation in warm and cold seasons. - Operator’s seat with suspension, adjustable by weight and position (longitudinal adjustment and backrest). - External rear‑view mirrors, windshield wiper, windshield washer, and interior cabin lighting must be provided. Safety and operational requirements: - The Machine must be equipped with front headlights, position lights, rear lights, turn indicators, audible horns, and a warning beacon. - The operator’s seat must be equipped with a safety belt. Controls must be arranged in a convenient and safe manner for the operator. - An identification plate must be attached to the Machine, indicating the Manufacturer’s name, model, serial number, year of manufacture, and main technical parameters. - Structural elements subject to corrosion must have anti‑corrosion protection (coatings, materials). - The design must provide accessibility of main components and assemblies for maintenance and repair. Completeness: - The delivery set must include: a set of standard spare brushes for intended operation, a standard set of tools and accessories (spare parts kit) provided by the Manufacturer, operation documentation, product passport (data sheet), and warranty documents. - All indicators on the control panel and operation documentation must be in Russian/English. |
| 6. | Procedure for delivery and acceptance of goods, services, and work results | - Acceptance shall be carried out by the Customer’s authorized representatives in terms of quantity and quality. The following shall be checked: integrity of packaging (if applicable), conformity of completeness with the contract requirements and enclosed documents, labeling, and operability of the main systems. - A test start‑up and functional check of the Machine shall be performed, including engine operation, control systems, sweeping and cleaning systems (brushes, suction duct, hopper lifting and unloading), as well as lighting and signaling systems. - Upon successful verification, the following documents shall be signed: delivery notes, acceptance certificate, and, if necessary, commissioning certificate. - In case of any discrepancies in quantity, completeness, or quality, a report shall be drawn up documenting the identified deficiencies. The Supplier shall, at its own expense, rectify the deficiencies, complete the missing components, or replace the Machine within the time specified in the contract. - The Supplier’s obligations for delivery of the Machine shall be considered fulfilled upon signing of the acceptance certificate by the authorized representatives of both Parties. |
| 7. | Requirements for the submission of technical and other documents to the Customer upon completion and handover of the works | - The Supplier shall provide the Customer with the following documents simultaneously with the delivery of the Machine: - Product passport / datasheet (if available) or an equivalent Manufacturer’s document. - Operation and maintenance manual for the Machine. - Operator’s manual (instructions) on safe operation. - Warranty card (certificate) specifying the warranty period and conditions. - Spare parts catalog (Part Book). - If available – service logbook containing the scheduled maintenance regulations and a list of authorized service centers. - Copies of documents confirming the Machine’s compliance with applicable technical regulations and standards (certificates / declarations of conformity, permits), certified by the Supplier’s signature and stamp. - Enclosed documents: delivery note/waybill, acceptance certificate, invoice (if applicable). - Commissioning act and Customer’s personnel training report (if such work is performed). |
| 8. | Warranty obligations | - The warranty period for the Machine shall be not less than 12 (twelve) months from the date of signing the acceptance certificate, or not less than 1,000 operating hours, whichever comes first. The specific warranty period shall be stated in the contract. - During the warranty period, the Supplier (or an authorized service organization) shall provide free rectification of any detected defects or malfunctions of the Machine caused by the Manufacturer/Supplier, including the cost of spare parts and labor. The response time for a specialist’s visit and elimination of the malfunction shall not exceed 10 (ten) working days from the date of written notification by the Customer, unless otherwise specified in the contract. - The Supplier shall provide informational and consulting support to the Customer on issues related to operation and maintenance of the Machine throughout the entire warranty period. - The presence of the Supplier’s service center within the territory of the Kyrgyz Republic is desirable. |