Terms of Reference

for the purchase of equipment for automated geomechanical monitoring (automatic inclinometers and piezometers)

| Item № | Requirement parameters for goods, works, services to be purchased (subject of the purchase) | Purchase of original automatic inclinometers and piezometers designed for automated monitoring of rock mass deformations and groundwater levels. |
| --- | --- | --- |
| 1 | Description | Automatic inclinometer and piezometer, total quantity ordered: total coverage of 1,100 linear meters. |
| 2 | Delivery location | Kumtor Mine Site. |
| 3 | Delivery deadline | Delivery is 30 business days from the date of signing the Service Agreement. |
| 4 | Procedure (sequence, stages) of works | According to the Terms of Reference. |
| 5 | Requirements for goods, services and works to be delivered | - Automatic inclinometers: Type: stationary automatic system (chain inclinometer). Required characteristics: Measurement accuracy: not less than ±0.1 mm/m. Measurement segment spacing: ≤ 0.5 m. Capability to operate in vertical boreholes with a depth of up to 100 m or more. Operating temperature range: -30°C to +50°C. - Piezometers: Type: vibrating (VWP) or digital, suitable for automatic data logging. Required characteristics: Accuracy: at least ±0.1% full scale. - Data collection and transmission system: - Capability to connect inclinometers and piezometers to a single automated monitoring cabinet (AMC). - Measurement interval: from 10 minutes (configurable). - Availability of software with: displacement profile plotting pore pressure variation graphs. |
| 6 | Completion and acceptance procedure | The Supplier shall provide: - Delivery of the equipment as a complete set. - User manuals in Russian. - Commissioning and setup. - Personnel training. - Warranty of at least 12 months. - Minimum delivery period. |
| 7 | Requirements for the submission of technical and related documentation to the Client upon completion and delivery of the work | Technical documentation from manufacturing companies, certificates, technical passports, documents certifying the quality of materials, products, and equipment used in production. |
| 8 | Warranty obligations | In case a defect or non-compliance with the specified parameters of the delivered goods is detected, the Supplier shall replace the goods or refund the payment. |