Terms of Reference

For purchasing High-pressure device (electric) for washing

| Item # | Specifications / Requirements for the procured Goods | Specific requirements for Goods |
| --- | --- | --- |
| 1 | Name and quantity | High-pressure washer (electric) Model (required): DEWALT DXPW 007 E (HRR) ET/25159 OEM №: DXPW007E Quantity: 8 units |
| 2 | Place of Goods delivery | Kyrgyz Republic, Balykchy town, Naryn Highway Street, 9. |
| 3 | Goods delivery terms (periods) | It is determined by the contract. Delivery includes delivery to the destination, unloading, and transfer of the goods to the Customer with a full set of documents. |
| 4 | Requirements for the procured Goods | Requirements for the condition of the Goods - The Goods must be new, not previously used, and not refurbished. - Supplied in the original factory packaging, with legible factory labels/nameplates and a serial number. - Year of manufacture — as recent as possible at the delivery time (preferably current or previous year). Key Technical Requirements (Mandatory) The supplied unit must comply with the specified model and have characteristics no worse than the following: - Drive type: electric, powered by AC mains. - Operating pressure: 230 bar - Capacity (flow rate): 840 l/h - Power supply: 380 V, 50 Hz, 3-phase (neutral/connection scheme as per the product manual, if required) - Power consumption/rated power: 7.4 kW - Field of application: industrial heavy duty washing suitable for Fuel-Oil Refinery conditions Safety and equipment requirements - Availability of standard safety features for proper operation: - pressure relief/bypass and/or safety valve (overpressure protection) - electric motor protection (thermal/current protection) or other standard protection as provided by the manufacturer. - ability to safely release pressure before disconnecting/disassembling the hose. - Electrical equipment must be rated as 380 V, 50 Hz power supply and have standard manufacturer‑approved connections according to the operating manual. Minimum Delivery Set (Required Completeness) The supplier must provide a full set ready for commissioning, including at minimum: - DEWALT DXPW007E (DXPW 007 E) high‑pressure washer - Spray gun – standard manufacturer model. - High-pressure hose – standard manufacturer model - lance and standard nozzles/tips (if included by the manufacturer) - power cable/connector (manufacturer version) or connection kit according to the product manual. - filter element/water inlet filter (if provided by the manufacturer) - packing list/delivery specification. |
| 5 | Procedure for Goods delivery and acceptance | - Acceptance is conducted after delivery to the production site and includes: - checking the integrity of the packaging and the absence of mechanical damage - verifying completeness according to the packing list / manufacturer’s specification - checking labeling (model DEWALT DXPW 007 E (HRR) ET/25159, OEM DXPW007E, serial number) - verifying the availability of documentation (datasheet/manual, operating instructions, warranty documents, certificates/compliance/import documents — where applicable) - performing a visual inspection and a test run/functionality check in a safe mode (within the capabilities of the Customer’s site and the manufacturer’s requirements), including checking for absence of leaks, correct operation of controls, and stable operation of the electric motor and pump. - Based on the results of the inspection at the Customer’s production site, an Acceptance Certificate / Handover Certificate is signed. |
| 6 | Requirements for providing Goods technical and other documentation to the Customer. | - Product passport / technical specification (datasheet) indicating the model and serial number. - Operation and safety manual (preferably in Russian; English is acceptable if it includes key safety and connection sections) - Warranty card / warranty terms - Documents confirming origin / legal import (for imported Goods) - Documents confirming compliance with EAEU requirements (if applicable to this product category) - Declaration / Certificate of Conformity to EAEU Technical Regulations for low‑voltage equipment and EMC, if required/applicable. |
| 7 | Warranty obligations | - Warranty: at least 12 months from the date of commissioning or delivery (whichever comes earlier - by agreement), or manufacturer’s warranty, if it is longer. - The warranty includes repair/replacement of defective components, provided that operating conditions are followed. |