Terms of Reference for the purchase of two diesel welding machines for HD Maintenance garage, Kumtor Gold Company CJSC

| Item | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1. | Delivery Location | KUMTOR GOLD COMPANY CJSC, 9 Naryn highway, Balykchy, Kyrgyz Republic. |
| 2. | Client | Kumtor Gold Company CJSC |
| 3. | General provisions | The subject of these Terms of Reference is the purchase of two units of diesel welding machines for field welders of HD Maintenance garage. |
| 4. | Requirements | - The welding machines supplied shall be new and unused. Purpose: welding operations in field conditions (mine open pit). The welding machines will be operated in high altitude conditions above 4,000 m above sea level, at the temperatures from - 35℃ to +40℃. - The equipment shall meet or exceed the requirements of the technical specifications in terms of performance and energy efficiency. - All necessary user manuals and technical documentation shall be included. - Submission of technical documentation, user manuals, or any other documents in copy form is not permitted. - The equipment shall comply with the quality requirements established by technical regulations, standardization documents, and state standards applicable to this type of Goods and in force in the territory of the Kyrgyz Republic, as well as with the EAEU standards. - The equipment shall comply with the safety requirements established by the applicable legislation. - When supplying the equipment, the Supplier shall provide originals or duly certified copies of valid certificates of conformity and declarations of conformity to the requirements of regulatory documents for the supplied Goods. - The equipment supplied shall not be pledged, under arrest or another encumbrance. |

| 6. | Technical Specifications: | 1. Model: | 1. Model: MILLER BIG BLUE 600 AIR PAK diesel welding machine or equivalent. 2. Year of manufacture: 2025 or later. 3. Welding power 3.1 Current range: 20-600 A. 3.2 Voltage: 15-50 V. 3.3 Rated output: - 500 A @ 40 V (100%) - 575 A @ 43 V (100%) - 600 A @ 44 V (60%) 4. Engine: Deutz TD2.9 L4 4.1 Power: 65.7 hp (≈49 kW), 4-cylinder inline. 4.2 Cooling: liquid. 4.3 Maximum torque: 260 Nm at 1,600–1,800 rpm. 4.4 Minimum idle speed: 900 rpm. 4.5 Fuel tank: 95 L. 5. Compressor (Air Pak) 5.1 Type: Ingersoll Rand screw compressor. 5.2 Capacity: ~60 CFM at 100 psi (100% duty cycle). 5.3 Function: separate on/off switch to save fuel and resources. 6. Generator (autonomous power supply) 6.1 3-phase output: 20 kW (continuous at 40 °C), up to 27 kW peak. 6.2 1-phase output: 12 kW (continuous), up to 15 kW peak. |
| --- | --- | --- | --- |
| 7. | Payment and delivery terms | - Payment terms: According to the agreement concluded. - The equipment shall be delivered by manufacturer to BMY within the deadlines specified by the Client. - Assembly and commissioning shall be completed within the deadlines specified by the Client. - In case of failure to comply with the deadlines, penalties shall apply. |  |
| 8. | Requirements for the submission of technical and other documents to the Client. | - Upon delivery of the Goods, the Supplier shall provide the Client with the following set of documents: - Consignment notes. - Invoice. - Original certificate of conformity. - Service book. - Equipment passport. - Documents confirming the manufacturer's and/or supplier's quality guarantee. - Documents certifying ownership of the vehicle, which serve as the basis for its state registration. - All necessary user manuals, technical documentation, and spare parts catalogs shall be included in the package or available in electronic form. |  |
| 9. | Warranty period | 1. The Supplier shall provide a warranty period for the purchased equipment in accordance with the Client’s request. |  |
| 10. | Commissioning | 1. All costs for additional work identified during the equipment inspection shall be borne by the Supplier. |  |
| 11. | Quality evaluation criteria | 1. After commissioning during the warranty period, the welding machines shall be evaluated. Evaluation results shall comply with the manufacturer's specifications. 2. In the event of non-compliance with the requirements, the Supplier shall bear all costs for its elimination. |  |
| 12. | Confidentiality | 1. Transaction data shall be confidential. The Contractor shall be liable for disclosure to third parties. |  |
| 13. | Environmental requirements | - The equipment shall comply with the environmental requirements established by the applicable laws. |  |
| 14. | Safety | - The equipment shall comply with the safety requirements established by the applicable laws. |  |