Приложение 1

Техническое задание

на поставку воздушного компрессора для нужд Подземной разработки местрождения «Кумтор» ЗАО «Кумтор Голд Компани»

| № п/п | Перечень основных данных и требований | Основные данные и требования |
| --- | --- | --- |
| 1. | Место доставки | ЗАО «КУМТОР ГОЛД КОМПАНИ». Кыргызская Республика, г. Балыкчы, ул. Нарынское шоссе, 9. |
| 2. | Заказчик | ЗАО «Кумтор Голд Компани», Подземная разработка |
| 3. | Общие положения | Целью настоящего технического задания является закупка воздушного компрессора для обеспечения подземных горных выработок сжатым воздухом, а также для использования в других производственных процессах, где требуется стабильное и бесперебойное снабжение сжатым воздухом. Компрессор должен обеспечить стабильную и безопасную работу в условиях подземных объектов при соблюдении всех требований безопасности и надежности. Компрессор должен быть предназначен для обеспечения сжатым воздухом различных систем и оборудования, таких как инструменты, системы вентиляции и водоотведения, транспортировки материалов и другие процессы, требующие подачи сжатого воздуха в подземных горных выработках, неопасных по газу и пыли. Компрессор должен быть новым, не бывшими в употреблении. Количество – 1 комплект. |
| 4. | Геометрические характеристики выработок | Максимальная высота: - Капитальные выработки - 5200 мм. - Эксплуатационные выработки – 4500 мм. Максимальная ширина: - Капитальные выработки - 5500 мм. - Эксплуатационные выработки – 4500 мм. Минимальный наружный радиус поворота, 6400 мм. Минимальный внутренний радиус поворота, 3300 мм. Максимальный уклон наклонного съезда, 15% Дорожное покрытие – порода. |
| 5. | Свойства окружающей среды | Высота над уровнем моря - 4000 м. Наружная температура воздуха от – 35 ℃ до +40 ℃ Температура воздуха шахты от +2 до +15 ℃ Относительная влажность воздуха, ~70% Водоснабжение – Техническая вода Загазованность – Нет Запыленность – Нет |
| 6. | Условия эксплуатации | Оборудование будет использоваться в подземных горных выработках, неопасных по газу и пыли. Рабочие условия включают повышенную влажность, перепады температур, наличие пыли и загрязняющих веществ. Применение в условиях возможных механических воздействий, вибрации и ограниченного пространства. |
| 7. | Требования к компрессору |  |
| 7.1. | Технические характеристики | Модель, марка и производитель: по результатам отбора. Тип компрессора: Воздушный компрессор с винтовым принципом работы. Оборудование должно обеспечивать стабильную и безопасную подачу сжатого воздуха, соответствующую условиям эксплуатации. Год выпуска: не ниже 2025 г. Производительность: Компрессор должен обеспечивать производительность от 30 до 40 м3/мин с возможностью регулировки в зависимости от рабочей нагрузки. Эта характеристика важна для обеспечения нормальной работы всех связанных систем. Давление сжатого воздуха: Рабочее давление компрессора должно варьироваться от 8 до 10 бар, в зависимости от требований, предъявляемых к системе сжатого воздуха на объекте. Мощность: Мощность компрессора должна соответствовать производительности, указанной выше, и обеспечивать его бесперебойную работу при нагрузках, характерных для подземных условий. Мощность оборудования будет уточняться в зависимости от выбранной модели и ее применения. Адаптация к работе в условиях высокогорья. Режим работы оборудования круглосуточный, продолжительность технологической смены 11 часов, 365 дней в году. |
| 7.2. | Требования к электрической системе | Напряжение – 380–390 В, 50–60 Гц. Общая потребляемая мощность - на рассмотрение. Компрессор должен быть оснащен системой электропитания, соответствующей стандартам и требованиям безопасности для подземного рудника. |
| 7.3. | Требования к размерам и конструкции | Габаритные размеры компрессора должны соответствовать следующим диапазонам: длина – не более 4 м, ширина не более – 3 м, высота не более – 3 м. с учетом ограничений пространства, существующих в подземных выработках. Компрессор должен иметь компактную конструкцию, обеспечивающую удобство транспортировки. Масса компрессора: на рассмотрение, При этом предложенная масса должна обеспечить удобство транспортировки и минимизирует нагрузку на инфраструктуру при его перемещении в условиях шахты. Конструкция компрессора должна быть разработана с учетом снижения уровня вибраций, так как это влияет на долговечность оборудования и его работу в условиях подземного рудника. |
| 7.4. | Требования к безопасности | Компрессор должен быть оборудован системой защиты от перегрева, которая автоматически отключает оборудование при достижении предельных температурных значений, чтобы предотвратить повреждения оборудования. Все металлические части компрессора, подверженные воздействию влаги и агрессивных условий, должны быть покрыты антикоррозийным материалом (например, горячее оцинкование или стойкое к коррозии покрытие). Компрессор должен быть оснащен системой аварийной сигнализации и безопасности (например, для сигнализации о низком уровне масла, перегреве, утечках и других неисправностях). Все элементы компрессора должны быть сертифицированы для безопасной эксплуатации в подземных горных выработках. Оборудование должно быть оснащено системой мониторинга состояния, позволяющей дистанционно отслеживать параметры работы компрессора, а также автоматически сигнализировать о возникших неисправностях или отклонениях от нормальных рабочих параметров. Компрессор должен иметь эффективную систему охлаждения, которая обеспечивает нормальную работу оборудования при подземных температурах и влажности. Возможность использования воздушного охлаждения. Уровень шума компрессора должен быть минимизирован и не превышать 80 дБ на расстоянии 1 метра от оборудования. Компрессор должен быть защищен от попадания пыли и других загрязняющих веществ. Все движущиеся части компрессора, и другие представляющие собой источники опасности для людей, должны быть ограждены, изолированы и защищены в соответствии с требованиями безопасности. |
| 7.4. | ЗИП | Поставщик должен поставить вместе с оборудованием комплект ЗИП и ремкомплект на 4000 и 8000 моточасов. Поставщик должен гарантировать наличие запасных частей для компрессора в течение как минимум 5 лет после поставки оборудования. |
| 8. | Гарантийный срок и обучение | Поставщик должен предоставить гарантийный срок на приобретаемое оборудование не менее 24 месяцев со дня ввода в эксплуатации. Наличие сервис центра и склада запасных частей на территории КР. В случае отсутствия сервисного центра и склада запасных частей на территории страны Заказчика Поставщик обязуется создать необходимые условия для организации сервисного обслуживания и технической поддержки в стране пребывания Заказчика. Поставщик вместе с оборудованием обязуется предоставить полную техническую документацию на компрессор (чертежи, сертификаты соответствия, технические паспорта, инструкции по эксплуатации, каталог запасных частей и другие необходимые документы для безопасной эксплуатации, а также для прохождения таможенного оформления и регистрации в государственных органах КР). Все документы должны быть предоставлены как в электронном виде, так и в бумажном на русском и английском языках. Оборудование должно эксплуатироваться круглосуточно (24/7). Поставщик должен провести обучение персонала Заказчика по эксплуатации и обслуживанию компрессора - на руднике Кумтор. Провести 24-часовое тестирование оборудования после пуско-наладочных работ с обязательным присутствием сервисного инженера поставщика. |
| 9. | Приемка оборудования | Приемка оборудования по качественным и количественным показателям производиться на территории рудника «Кумтор» с участием представителя Поставщика. |
| 10. | Запуск | Все затраты за дополнительные работы выявленные в ходе проверки оборудования несет Поставщик. |
| 11. | Критерии оценки качества | После ввода в эксплуатацию в течение гарантийного срока производится оценка оборудования. Характеристика должна соответствовать заводским показаниям. В случае выявления несоответствий наших требований Поставщик берет на себя все затраты по их устранению. |
| 12. | Сроки поставки | Доставка осуществляется за счет и силами Поставщика. Условия поставки: для нерезидентов КР- DAP, для резидентов КР -DDP. Место доставки: Кыргызская Республика, г. Балыкчы, ул. Нарынское шоссе, 9. Срок поставки: до 180 календарных дней с момента подписания Договора Сторонами. Сборка и ввод в эксплуатацию в установленные сроки Заказчиком. |
| 13. | Технические регламенты и стандарты | Оборудование должно соответствовать требованиям, установленным действующим Техническим регламентом Таможенного союза (ТР ТС 010/2011) «О безопасности машин и оборудования» и другими действующими требованиями ЕАЭС (при необходимости). |
| 14. | Примечание | Требования, указанные в настоящем ТЗ, является ориентировочным и может быть скорректировано в процессе обсуждения с потенциальными поставщиками. |

Appendix No. 1

Terms of Reference

for the supply of an air compressor for the Underground Mining Operations

of the Kumtor Deposit of CJSC Kumtor Gold Company

| № п/п | Description of Key Data and Requirements | Key Data and Requirements |
| --- | --- | --- |
| 1. | Delivery Location | CJSC Kumtor Gold Company, Kyrgyz Republic, Balykchy City, 9 Naryn Highway |
| 2. | Client | CJSC Kumtor Gold Company, Underground Mining |
| 3. | General Provisions | The purpose of this Terms of Reference is to procure an air compressor to supply compressed air for underground mine workings, as well as for use in other production processes requiring stable and uninterrupted compressed air supply. The compressor shall ensure reliable and safe operation under underground conditions in compliance with all safety and reliability requirements. The compressor shall be designed to supply compressed air to various systems and equipment, including tools, ventilation systems, dewatering systems, material handling, and other processes requiring compressed air in underground mine workings that are non–gas-hazardous and non-dust-hazardous. The compressor shall be new, and not previously operated. Quantity: 1 set. |
| 4. | Geometrical Characteristics of Mine Workings | Maximum height: • Capital workings – 5200 mm • Production workings – 4500 mm Maximum width: • Capital workings – 5500 mm • Production workings – 4500 mm Minimum external turning radius – 6400 mm Minimum internal turning radius – 3300 mm Maximum degree of decline ramp – 15% Road surface – rock |
| 5. | Environmental Conditions | Elevation above sea level – 4000 m Ambient air temperature –35 °C to +40 °C Mine air temperature +2 °C to +15 °C Relative humidity – approx. 70% Water supply – technical water Gas hazards – None Dust hazards – None |
| 6. | Operating Conditions | The equipment will be operated in underground mine workings that are non-hazardous in terms of gas and dust. Operating conditions include increased humidity, temperature fluctuations, presence of dust and contaminants. Operation under potential mechanical impacts, vibration, and limited space. |
| 7. | Compressor Requirements |  |
| 7.1. | Technical Characteristics | Model, brand, and manufacturer – to be determined based on selection results. Compressor type – rotary screw air compressor. The equipment shall ensure stable and safe compressed air supply suitable for underground operating conditions. Year of manufacture – not earlier than 2025. Capacity: The compressor shall provide a capacity of 30 to 40 m³/min, with the ability to regulate output depending on operational load. This parameter is critical to ensure proper and stable operation of all associated systems. Compressed Air Pressure: The operating pressure of the compressor shall range from 8 to 10 bar, depending on the requirements of the compressed air system at the site. Power Rating: The compressor power rating shall correspond to the capacity specified above and ensure continuous and reliable operation under load conditions typical for underground mining operations. The final power rating shall be specified depending on the selected model and its intended application. The compressor shall be adapted for operation under high-altitude conditions. The equipment shall operate on a continuous basis (24/7), with a technological shift duration of 11 hours, 365 days per year. |
| 7.2. | Electrical System Requirements | Voltage: 380–390 V, 50–60 Hz. Total Power Consumption: subject to review. The compressor shall be equipped with an electrical power supply system that fully complies with applicable standards and safety requirements for operation in an underground mine environment. |
| 7.3. | Dimensions and Design Requirements | The overall dimensions of the compressor shall not exceed the following limits, taking into account the spatial constraints of underground mine workings: length: not more than 4 m, width: not more than 3 m, height: not more than 3 m. The compressor shall have a compact design to ensure ease of transportation and handling in underground conditions. Масса компрессора: на рассмотрение, При этом предложенная масса должна обеспечить удобство транспортировки и минимизирует нагрузку на инфраструктуру при его перемещении в условиях шахты. Конструкция компрессора должна быть разработана с учетом снижения уровня вибраций, так как это влияет на долговечность оборудования и его работу в условиях подземного рудника. Compressor weight: subject to review. The proposed weight shall ensure convenient transportation and minimize load on underground infrastructure during movement within the mine. The compressor design shall incorporate vibration reduction measures, as reduced vibration is critical to equipment durability and reliable operation in an underground mining environment. |
| 7.4. | Safety Requirements | The compressor shall be equipped with an overheating protection system that automatically shuts down the equipment when critical temperature limits are reached, in order to prevent damage to the compressor. All metallic parts of the compressor exposed to moisture and aggressive environmental conditions shall be protected by anti-corrosion treatment (e.g. hot-dip galvanizing or corrosion-resistant coating). The compressor shall be equipped with an emergency alarm and safety system, including alarms for low oil level, overheating, leaks, and other malfunctions. All compressor components shall be certified for safe operation in underground mine workings. The equipment shall be equipped with a condition monitoring system enabling remote monitoring of compressor operating parameters and automatic alarming in case of failures or deviations from normal operating conditions. The compressor shall be fitted with an efficient cooling system ensuring normal operation under underground temperature and humidity conditions. The use of air cooling shall be acceptable. The compressor noise level shall be minimized and shall not exceed 80 dB at a distance of 1 meter from the equipment. The compressor shall be protected against ingress of dust and other contaminants. All moving parts of the compressor and any other components posing potential hazards to personnel shall be guarded, isolated, and protected in full compliance with applicable safety requirements. |
| 7.4. | Spare Parts (SP Kit) | The Supplier shall deliver, together with the equipment, a spare parts kit and maintenance kit designed for 4,000 and 8,000 operating hours. The Supplier shall guarantee the availability of spare parts for the compressor for a minimum period of five (5) years following delivery of the equipment. |
| 8. | Warranty and Training | The Supplier shall provide a warranty period of not less than twenty-four (24) months from the date of commissioning of the supplied equipment. Availability of a service center and a spare parts warehouse within the territory of the Kyrgyz Republic is required. In the absence of a service center and spare parts warehouse within the Client’s country, the Supplier shall undertake to establish the necessary conditions for organizing service maintenance and technical support within the country of the Client. Together with the equipment, the Supplier shall provide complete technical documentation for the compressor, including but not limited to drawings, certificates of conformity, technical data sheets, operation and maintenance manuals, spare parts catalogues, and any other documentation required for safe operation, customs clearance, and registration with the state authorities of the Kyrgyz Republic. All documentation shall be provided both in electronic and hard-copy formats, in Russian and English. The equipment shall be designed for continuous operation (24/7). The Supplier shall conduct training for the Client’s personnel on compressor operation and maintenance at Kumtor Mine. The Supplier shall also perform 24-hour performance testing of the equipment after commissioning, with mandatory presence of the Supplier’s service engineer. |
| 9. | Equipment Acceptance | Acceptance of the equipment in terms of quantity and quality shall be carried out at the Kumtor Mine, with the participation of the Supplier’s authorized representative. |
| 10. | Commissioning | All costs related to any additional works identified during inspection and commissioning of the equipment shall be borne by the Supplier. |
| 11. | Quality Evaluation Criteria | After commissioning, the equipment shall be subject to performance evaluation throughout the warranty period. The equipment performance parameters shall fully comply with the manufacturer’s rated specifications. In the event of any non-compliance with the stated requirements, the Supplier shall bear all costs associated with corrective actions and remediation. |
| 12. | Delivery Schedule | Delivery of the equipment shall be performed at the Supplier’s cost and responsibility. Delivery terms: DAP – for non-residents of the Kyrgyz Republic; DDP – for residents of the Kyrgyz Republic. Delivery location: Kyrgyz Republic, Balykchy, 9 Naryn Highway. Delivery time: up to 180 calendar days from the date of signing of the Contract by both Parties. Assembly and commissioning shall be completed within the deadlines agreed with the Client. |
| 13. | Technical Regulations and Standards | The equipment shall comply with the requirements of the applicable Technical Regulation of the Customs Union TR CU 010/2011 “On Safety of Machinery and Equipment”, as well as with other applicable EAEU regulations, where required. |
| 14. | Note | The requirements specified in this Terms of Reference are indicative and may be amended or refined during discussions with potential suppliers. |