ТЕХНИЧЕСКОЕ ЗАДАНИЕ

на закуп «15-тонного вилочного погрузчика HELI или эквивалент»

| № п/п | Параметры требований закупаемых товаров, работ, услуг (предмет закупки) | Конкретные требования к товарам, работам, услугам |
| --- | --- | --- |
| 1. | Наименование/вид количество /объемы единица измерения | Вилочный погрузчик дизельный фронтальный. Грузоподъёмность: 15000 кг. Количество: 1 (одна) единица. Единица измерения: штука (шт.). |
| 2. | Место поставки товара, оказания услуг/выполнения работ | Балыкчинская перевалочная база «Кумтор», г. Балыкчы Нарынское шоссе №9. |
| 3. | Сроки (периоды) поставки товара, оказания услуг, выполнения работ | Предусмотреть в договоре. Поставка осуществляется единоразово, полной комплектностью. |
| 4. | Порядок (последовательность, этапы) выполнения работ | - Согласование с Заказчиком даты и времени поставки, порядка разгрузки и места. - Доставка погрузчика Поставщиком до места поставки. - Разгрузка и установка погрузчика на площадке Заказчика. - Проведение пусконаладочных работ (проверка работоспособности всех систем, контрольных и сигнальных устройств). - Передача Заказчику комплекта технической документации, подписание акта приёмки-передачи. |
| 5. | Требования к поставляемым товарам, выполняемым работам, оказываемым услугам | - Погрузчик новый, не бывший в эксплуатации, не восстановленный. - Тип: вилочный погрузчик дизельный, фронтальный. - Двигатель внутреннего сгорания: дизельный. - Мощность ДВС: не менее 140 кВт. - Грузоподъёмность: не менее 15000 кг. - Высота подъёма вил: не менее 3000 мм. - Тип мачты: двухсекционная мачта с полным свободным ходом (Two-stage full free mast). - Длина вил: не менее 2400 мм. - Позиционер вил: с автоматическим смещением вил (автоматический позиционер вил). - Коробка переключения передач: автоматическая. - Погрузчик должен быть полностью укомплектован и подготовлен к эксплуатации: заправлен необходимыми эксплуатационными жидкостями (топливо, масла, охлаждающая жидкость и др.), отрегулирован и проверен Поставщиком. - Наличие кабины оператора с рабочим и ходовым освещением, звуковым сигналом, зеркалами заднего вида, ремнём безопасности, устройством звуковой/световой сигнализации движения задним ходом. - Наличие счётчика моточасов, приборов контроля температуры, давления масла, уровня топлива и других, предусмотренных заводской компоновкой. - Наличие заводской таблички (шильдика) с указанием модели, серийного номера, года выпуска, основных технических характеристик и грузовой диаграммы. |
| 6. | Порядок сдачи и приемки товаров, услуг, результатов работ | - Приёмка осуществляется Заказчиком по количеству, комплектности, качеству и соответствию требованиям настоящего технического задания. - Приёмка по количеству и комплектности – по товаросопроводительным документам Поставщика (товарная накладная/спецификация). - Приёмка по качеству – путём визуального осмотра, проверки целостности, соответствия маркировки и серийных номеров, а также путём проверки работоспособности (запуск двигателя, проверка систем управления, движения, подъёма/опускания мачты, работы осветительных и сигнальных приборов). - Результаты приёмки оформляются актом приёмки-передачи, подписанным уполномоченными представителями Сторон. - В случае выявления несоответствий (дефектов, недокомплектности) составляется двусторонний акт с перечнем замечаний. - Поставщик обязан устранить выявленные недостатки или произвести замену товара за свой счёт в согласованный срок. |
| 7. | Требования по передаче заказчику технических и иных документов по завершению и сдаче работ | Одновременно с поставкой и сдачей товара Поставщик передаёт Заказчику: - Паспорт (формуляр) изделия на каждый погрузчик. - Руководство по эксплуатации (инструкция по эксплуатации и техническому обслуживанию) на русском языке – не менее 1 экз. в бумажном виде (и/или электронном по согласованию). - Каталог (перечень) запасных частей и расходных материалов (бумажный или электронный вариант). - Гарантийный талон (сертификат) производителя/Поставщика с указанием сроков и условий гарантии. - Сертификаты соответствия/декларации о соответствии требованиям технических регламентов (при наличии обязательной сертификации). - Товаросопроводительные документы: счёт, товарная накладная, спецификация, акт приёмки-передачи. - Иные документы, предусмотренные законодательством и документацией на данную модель (при наличии). |
| 8. | Гарантийные обязательства | - Гарантийный срок эксплуатации погрузчика – не менее 12 (двенадцати) месяцев или 2000 моточасов (в зависимости от того, что наступит ранее) с даты подписания акта приёмки-передачи/акта ввода в эксплуатацию. - В течение гарантийного срока Поставщик обеспечивает бесплатное устранение производственных и монтажных дефектов, включая стоимость запасных частей, работ и выезда специалиста. - Максимальный срок реагирования на заявку Заказчика о гарантийной неисправности – не более 3 (трёх) рабочих дней; срок устранения неисправности – не более 10 (десяти) рабочих дней, если не требуется поставка редких узлов/агрегатов (в этом случае срок согласуется сторонами отдельно). - На заменённые по гарантии узлы и агрегаты предоставляется гарантия не менее оставшегося общего гарантийного срока на изделие (если иное не установлено производителем/договором). |

TECHNICAL SPECIFICATION

for Procurement of a 15-Ton Forklift Truck HELI or Equivalent

| No. | Parameters of Requirements for Procured Goods, Works, Services | Specific Requirements for Goods, Works, Services |
| --- | --- | --- |
| 1 | Name / Type, Quantity / Volume, Unit of Measurement | Diesel front forklift truck. Rated load capacity: 15,000 kg. Quantity: 1 (one) unit. Unit of measurement: piece (pcs). |
| 2 | Place of Supply of Goods / Provision of Services / Performance of Works | Kumtor Balykchy Transshipment Base, 9 Narynskoye Highway, Balykchy city. |
| 3 | Delivery Timeframes (Periods) | To be specified in the Contract. Delivery shall be made in one batch, fully complete. |
| 4 | Procedure (Sequence, Stages) of Works Performance | • Coordination with the Customer of delivery date and time, unloading procedure, and location. • Delivery of the forklift by the Supplier to the place of delivery. • Unloading and positioning of the forklift at the Customer’s site. • Commissioning works (verification of operability of all systems, control and warning devices). • Handover of the complete set of technical documentation to the Customer and signing of the Hand-Over Acceptance Certificate. |
| 5 | Requirements for Supplied Goods, Performed Works, Provided Services | • The forklift must be new, unused, and not refurbished. • Type: diesel front forklift truck. • Internal combustion engine: diesel. • Engine power: not less than 140 kW. • Rated load capacity: not less than 15,000 kg. • Fork lifting height: not less than 3,000 mm. • Mast type: two-stage full free lift mast. • Fork length: not less than 2,400 mm. • Fork positioner with automatic fork shifting. • Transmission: automatic. • The forklift must be fully equipped and ready for operation: filled with all necessary operating fluids (fuel, oils, coolant, etc.), adjusted and tested by the Supplier. • Operator’s cab equipped with working and travel lighting, audible alarm, rear-view mirrors, seat belt, and audible/visual reverse warning system. • Availability of hour meter, gauges for temperature, oil pressure, fuel level, and other instruments предусмотренные заводской компоновкой (as per manufacturer configuration). • Presence of manufacturer’s nameplate (ID plate) indicating model, serial number, year of manufacture, key technical parameters, and load chart. |
| 6 | Procedure for Acceptance and Inspection of Goods, Services, Work Results | • Acceptance is performed by the Customer based on quantity, completeness, quality, and compliance with this Technical Specification. • Acceptance by quantity and completeness – according to Supplier’s shipping documents (delivery note/specification). • Acceptance by quality – through visual inspection, verification of integrity, conformity of markings and serial numbers, and functional testing (engine start, control systems, travel, mast lifting/lowering, lighting and warning devices). • Acceptance results are documented in a Hand-Over Acceptance Certificate signed by authorized representatives of both Parties. • In case of non-conformities (defects, incompleteness), a bilateral inspection report listing all remarks shall be prepared. • The Supplier shall eliminate all identified deficiencies or replace the goods at its own expense within the agreed timeframe. |
| 7 | Requirements for Transfer of Technical and Other Documentation upon Completion and Acceptance | Together with delivery and acceptance of the goods, the Supplier shall provide the Customer with: • Product passport (manufacturer’s data sheet) for each forklift. • Operation and maintenance manual in Russian – at least 1 hard copy (and/or electronic version upon agreement). • Spare parts and consumables catalog/list (hard copy or electronic). • Warranty certificate issued by the manufacturer/Supplier with warranty terms and conditions. • Certificates of conformity / declarations of conformity with technical regulations (where mandatory certification applies). • Shipping documents: invoice, delivery note, specification, hand-over acceptance certificate. • Other documents required by applicable legislation and model documentation (if any). |
| 8 | Warranty Obligations | • Warranty period: not less than 12 (twelve) months or 2,000 operating hours (whichever occurs first), starting from the date of signing the Acceptance or Commissioning Certificate. • During the warranty period, the Supplier shall provide free elimination of manufacturing and installation defects, including cost of spare parts, labor, and specialist travel. • Maximum response time to a warranty claim: not more than 3 (three) working days; defect elimination period: not more than 10 (ten) working days, unless delivery of rare components is required (in such cases, срок согласуется сторонами separately). • Replaced units and components shall be covered by a warranty period not less than the remaining general warranty period for the forklift, unless otherwise established by the manufacturer or the contract. |