TERMS OF REFERENCE

for the purchase of a Twin-Shaft Shredder for PET plastic bottles

| Item № | Requirement parameters for goods, works, services to be purchased (subject of the purchase) | Specific requirements for goods, work, and services |
| --- | --- | --- |
| 1 | Name/type/quantity/volumes/measure unit | - Description: Twin-Shaft Shredder. - Quantity: 1 pc. - Measure unit: pc. - Technical specifications: Shredding chamber dimensions: 400 × 430 mm. Motor power: 7.5 kW (380 V). Feed hopper dimensions – minimum: 690 × 720 × 600 mm, maximum: 1,000 × 1,000 × 600 mm. Number of motors: 1 pc. Number of gearboxes: 1 pc. Shredder type: twin-shaft with disc cutters. Number of shafts: 2 pcs. Number of cutters: to be selected based on the raw material, required particle size, and required throughput. Thickness of cutters: to be selected based on the raw material, required particle size, and required throughput. Number of teeth per cutter: to be selected based on the raw material, required particle size, and required throughput. Cutter material: steel 45 with heat treatment. Output particle size: up to 100 mm (depends on cutter parameters). Equipment package: removable feed hopper, cable, and power connector. |
| 2 | Delivery location | Food Storage Facility, Kumtor Mine, Jety-Oguz district, Issyk-Kul region, Kyrgyz Republic. |
| 3 | Delivery deadline | The equipment shall be delivered no later than 90 calendar days from the date of signing the Agreement or receiving an advance payment. |
| 4 | Procedure (sequence, stages) of works | The equipment shall be delivered in a single stage. The delivery shall be completed in full before the expiry of the specified period. |
| 5 | Requirements for goods, services and works to be delivered | - The equipment shall be new and not used. - Certificates of conformity, passport or technical description shall be attached. - Packaging providing protection against moisture and mechanical damage during transportation shall be ensured. - The Supplier shall be responsible for the safety of the goods until they are delivered to the Client. - Upon delivery, an Acceptance Certificate shall be provided. - Return or replacement in case of a defect shall be ensured. |
| 6 | Completion and acceptance procedure | To be determined by Procurement Department. |
| 7 | Requirements for the submission of technical and related documentation to the Client upon completion and delivery of the work | Requirements regarding other documents shall be determined by Procurement Department. |
| 8 | Warranty obligations | Warranty obligations: The Supplier shall provide a warranty of at least 12 months from the date of commissioning or 18 months from the date of delivery. During this period, the Supplier shall, at the first request of the Client, replace free of charge any item found to be defective due to the manufacturer’s fault. The warranty shall not cover cases of improper operation, improper storage conditions, mechanical damage, or attempted repairs. The item shall be accompanied by technical documentation confirming compliance with the stated specifications. |