Terms of Reference

for the purchase of hoists

| Item № | Required parameters for goods, works, services to be purchased (subject of the purchase) | Specific requirements for goods, work, and services |
| --- | --- | --- |
| 1 | Name/type/quantity/volumes/measure unit | Supply of hoists: - 2 units with a 3-ton lifting capacity. - 1 unit with a 2-ton lifting capacity. |
| 2 | Delivery location | Kumtor Gold Mine, Jeti-Oguz District, Issyk-Kul Region. |
| 3 | Delivery deadline | Delivery deadline: no more than 90 calendar days from the date of the agreement conclusion. |
| 4 | Procedure (sequence, stages) of work | Single-stage delivery, subject to prior agreement. The Supplier shall submit a Commercial Proposal based on the specified parameters. |
| 5 | Requirements for goods, services and works to be delivered | 1. General requirements: - Type: electric hoist, rope or chain type. - Short-term overload during lifting: up to 125% of the rated capacity. - Power supply: 3-phase, 380 V, 50 Hz, without neutral. - Travel path: straight monorail. - Control: pendant push-button control with emergency stop. - Operating conditions: altitude > 4,000 m above sea level. - Temperature range: −40 to +20 °C. - Installation type: on an H-beam (monorail). Electric hoist for Area 1 - Hoist power supply via a 4-meter straight busbar system. - Rated lifting capacity: 3 tons. - Hook lifting height: 12 m. - Environment: aggressive, acidic. - Protection degree: not lower than IP64 for electrical equipment (motors, terminal box, control pendant). - Travel along the beam: motorized trolley with forward/reverse electric drive. - Power supply: cable. - Mounting: two-point attachment to the I-beam shelves. Beam dimensions: - Height: 360 mm. - Shelf width: 145 mm. - Shelf thickness: 15 mm. Beam standard: - S15 x 42.9 according to ASTM A6 (S-shapes) / GOST R 58966-2020 / I-beam 36M according to GOST 19425-74. Electric hoist for Area 2 - Flexible power cable for hoist supply. - Rated lifting capacity: 3 tons. - Hook lifting height: 10 m. - Environment: dusty. - Protection degree: not lower than IP64. - Travel along the beam: motorized trolley with forward/reverse electric drive. - Mounting: two-point attachment to the I-beam shelves. - Power supply: a trolley power supply from the previously installed hoist is available in the installation area. Beam dimensions: - Height: 360 mm. - Shelf width: 130 mm. - Shelf thickness: 15 mm. Beam standard: - S15 x 42.9 according to ASTM A6 (S-shapes) / GOST R 58966-2020 / I-beam 36M according to GOST 19425-74. Electric hoist for Area 3 - Flexible power cable for hoist supply. - Rated lifting capacity: 2 tons. - Hook lifting height: 12 m. - Environment: open air. - Protection degree: not lower than IP64, outdoor design. - Travel along the beam: non-driven trolley, manual idle carriage. - Power supply: cable. Beam dimensions: - Height: 360 mm. - Shelf width: 180 mm. - Shelf thickness: 15 mm. Beam standard: - S15 x 42.9 according to ASTM A6 (S-shapes) / GOST R 58966-2020 / I-beam 36M according to GOST 19425-74. |
| 6 | Completion and acceptance procedure | The Supplier shall provide the Client with: - Product passport. - Operation manual. - Spare parts: set of consumables for 12 months of operation (the list shall be included in the Commercial Proposal). |
| 7 | Requirements for the submission of technical and related documentation to the Client upon completion and delivery of the work | Copies of the following documents shall be submitted: certificate of conformity/origin and manufacturer’s warranty terms. |
| 8 | Warranty obligations | The Supplier shall provide a warranty of at least 12 months from the date of the equipment installation. In case of manufacturing defects identified during the warranty period, the equipment shall be replaced free of charge. |