Appendix 3 to the Procurement Organization and Implementation Policy of Kumtor Gold Company CJSC

APPROVED BY:

Full Name _______________________

Structural Subdivision Head

(in accordance with the Delegation of Financial Authorities Policy)

_________________________, 2025

Terms of Reference

Terms of Reference for the purchase of hoists

| Item № | Requirement parameters for the Goods/Services to be purchased | Basic data and requirements |
| --- | --- | --- |
| 1 | Name/type/quantity/volumes/measure unit | Supply of hoists: - 2 units with a 3-ton lifting capacity. |
| 2 | Delivery location | Kumtor Gold Mine, Jeti-Oguz District, Issyk-Kul Region. |
| 3 | Delivery deadline | Delivery deadline: no more than 90 calendar days from the date of the agreement conclusion. |
| 4 | Requirements for goods, services and works to be delivered | 1. General requirements: - Type: electric hoist, rope or chain type. - Short-term overload during lifting: up to 125% of the rated capacity. - Power supply: 3-phase, 380 V, 50 Hz, without neutral. - Travel path: straight monorail. - Control: pendant push-button control with emergency stop. - Operating conditions: altitude > 4,000 m above sea level. - Temperature range: −40 to +20 °C. - Installation type: on an H-beam (monorail). Hoist for Area 1. - Rated lifting capacity: 3 tons. - Hook lifting height: 12 m. - Environment: aggressive, saline. - Protection degree: not lower than IP64 for electrical equipment (motors, terminal box, control pendant). - Travel along the beam: motorized trolley with forward/reverse electric drive. - Power supply: cable. - Mounting: two-point attachment to the I-beam shelves. Beam dimensions: - Height: 360 mm. - Shelf width: 145 mm. - Shelf thickness: 15 mm. Conditions: - Temperature range: constant -40 to +20 °C. - Under a canopy. Designed: - For handling corrosive materials. Hoist for Area 2. - Rated lifting capacity: 3 tons. - Hook lifting height: 10 m. - Environment: dusty. - Protection degree: not lower than IP44. - Travel along the beam: motorized trolley with forward/reverse electric drive. - Mounting: two-point attachment to the I-beam shelves. - Power supply: a trolley power supply from the previously installed hoist is available in the installation area. Beam dimensions: - Height: 360 mm. - Shelf width: 130 mm. - Shelf thickness: 15 mm. Conditions: - Temperature range: constant +15/20 °C. - Enclosed indoor area with high dust levels. Designed: - For equipment maintenance. |
| 5 | Completion and acceptance procedure | The Supplier shall provide the Client with: - Product passport. - Operation manual. - Spare parts: set of consumables for 12 months of operation (the list shall be included in the commercial proposal). |
| 6 | Warranty obligations | The Supplier shall provide a warranty of at least 12 months from the date of the equipment installation. In case of manufacturing defects identified during the warranty period, the equipment shall be replaced free of charge. |

Drafted by:

_______________________                                      _________________________

Position	Full name

_______________________                                      ________________________

Position	Full name