Terms of Reference for the purchase of

one unit of a diesel mobile heater for HD Maintenance garage,

Kumtor Gold Company CJSC

| Item № | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1. | Delivery location | KUMTOR GOLD COMPANY CJSC, 9 Naryn highway, Balykchy, Kyrgyz Republic. |
| 2. | Client | Kumtor Gold Company CJSC |
| 3. | General provisions | The subject of these Terms of Reference is the purchase of one unit of a diesel mobile heater for HD Maintenance garage. |
| 4. | Requirements for the heater | - The supplied heater shall be new and unused. The purpose is to provide heating for mobile repair facilities (domes), as well as to warm up frozen equipment. The heater will be operated in high altitude conditions above 4,000 m above sea level, at the temperatures from - 35℃ to +40℃. - The equipment shall meet or exceed the requirements of the technical specifications in terms of performance and energy efficiency. - All necessary user manuals and technical documentation shall be included. - Technical documentation, user manuals, or any other documents in copy form shall not be accepted. - The equipment shall comply with the quality requirements established by technical regulations, standardization documents, and state standards applicable to this type of Goods and in force in the territory of the Kyrgyz Republic, as well as with the EAEU standards. - The equipment shall comply with the safety requirements established by the applicable legislation. - When supplying the equipment, the Supplier shall provide originals or duly certified copies of valid certificates of conformity and declarations of conformity to the requirements of regulatory documents for the supplied Goods. - The equipment supplied shall not be pledged, under arrest or another encumbrance. |

| 6. | Technical specifications: | 1. Model: | 1. Model: Diesel mobile heater ALLMAND MAXI-HEAT MH-1000 or equivalent. 2. Year of manufacture: 2025 or later. 3. Performance and heating 3.1 Heating capacity: 1,000,000 BTU/h (approximately 293 kW), provided by two independent heaters of 500,000 BTU each. 3.2 Airflow: ≈ 6,200 CFM (~10,550 m³/h). 3.3 Temperature rise: up to 85°C (185°F) in the heating zone. 4. Engine and generator 4.1 Model: Caterpillar C1.5. 4.2 Operating volume: 1,496 cm³ (1.5 L). 4.3 Rotation speed: 1,800 rpm. 4.4 Type: 4-stroke, inline 3-cylinder diesel engine, liquid-cooled, with indirect injection. 4.5 Cylinder diameter × stroke: 84 mm × 90 mm. 5. Generator 5.1 Model: Mecc Alte. 5.2 Generator: 9 kW, 1‑phase, 60 Hz, transformer controlled. 6. Design and options 6.1 iQ system: automatic burner calibration based on temperature, humidity, and pressure. 6.2 Air recirculation: built-in, improves efficiency and reduces fuel consumption. 6.3 Duct system: standard 16″ flange, optional 12″ flange, compatible with flexible duct hoses, static pressure of 2.3″ wc allows duct runs up to 33 m (with 16″) or 15 m (with 12″). 6.4 Safety systems: automatic shutdown in case of low oil pressure or overheating. 7. Chassis and controls 7.1 Trolley: welded steel frame with jacks, hitch (adjustable 2″ ball or 3″ pintle ring), 5,000 lbs (approx. 2,268 kg) leaf spring axle, 15″ tires, brakes (hydraulic or electric optional), lights, brake lights, turn signals, and safety chains. 7.2 Body: heavy-duty steel housing, lockable doors, internal lighting, ducting mounting points, toolbox. |
| --- | --- | --- | --- |
| 7. | Payment and delivery terms | - Payment terms: According to the agreement concluded. - The equipment shall be delivered by manufacturer to BMY within the deadlines specified by the Client. - Assembly and commissioning shall be completed within the deadlines specified by the Client. - In case of failure to comply with the deadlines, penalties shall apply. |  |
| 8. | Requirements for the submission of technical and other documents to the Client. | - Upon delivery of the Goods, the Supplier shall provide the Client with the following set of documents: - Consignment notes. - Invoice. - Original certificate of conformity. - Service book. - Equipment passport. - Documents confirming the manufacturer's and/or supplier's quality guarantee. - Documents certifying ownership of the equipment, which serve as the basis for its state registration. - All necessary user manuals, technical documentation, and spare parts catalogs shall be included in the package or available in electronic form. |  |
| 9. | Warranty period | 1. The Supplier shall provide a warranty period for the purchased equipment in accordance with the manufacturer's terms and conditions. |  |
| 10. | Commissioning | 1. All costs for additional work identified during the equipment inspection shall be borne by the Supplier. |  |
| 11. | Quality evaluation criteria | 1. After commissioning, the mobile heater shall be evaluated during the warranty period. Evaluation results shall comply with the manufacturer's specifications. 2. In the event of non-compliance with the requirements, the Supplier shall bear all costs for its elimination. |  |
| 12. | Confidentiality | 1. Transaction data shall be confidential. The Contractor shall be liable for disclosure to third parties. |  |
| 13 | Environmental requirements | - The equipment shall comply with the environmental requirements established by the applicable laws. |  |
| 14. | Safety | - The equipment shall comply with the safety requirements established by the applicable laws. |  |