TERMS OF REFERENCE

for the purchase of a crew bus for Kumtor Mine, Kumtor Gold Company CJSC

| Item № | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1. | Delivery location | 9 Naryn highway, Balykchy, Kyrgyz Republic. |
| 2. | Client | Kumtor Gold Company CJSC. |
| 3. | Contractor | To be determined based on the Bidding results. |
| 4. | General provision | The subject of these Terms of Reference is the purchase of four units of NEFAZ-4208 crew buses mounted on KAMAZ-43114 chassis or equivalent. |
| 5. | Requirements for the vehicle | - The supplied vehicle (hereinafter referred to as the vehicle) shall be new and unused. Type: off-road vehicle, classified as a Category D vehicle. Purpose: operation on public roads for people transportation. Operation: on paved and unpaved roads at temperatures ranging from –35 °C to +40 °C, at altitudes of up to 4,000 meters above sea level. - The vehicle shall meet or exceed the requirements of the technical specifications in terms of performance and energy efficiency. - All required user manuals and technical documentation shall be included in the package and shall not be accepted in the form of copies. - The vehicle shall comply with the quality requirements established by technical regulations, standardization documents, and state standards applicable to this type of Goods in the territory of the Kyrgyz Republic. - The vehicle shall comply with the safety requirements established by the applicable legislation. - When supplying the vehicle, the Supplier shall provide originals or duly certified copies of valid certificates of conformity and declarations of conformity to the requirements of regulatory documents for the supplied Goods. - The supplied vehicle shall not be pledged, under arrest or another encumbrance. |
| 6. | Technical specifications | The required technical specifications are listed in Appendix 1 (four pages). |
| 7. | Payment and delivery terms | - Payment terms: to be agreed with the Client. - The manufacturer shall deliver the vehicle to BMY within the deadlines specified by the Client. - Assembly and commissioning shall be completed within the deadlines specified by the Client. - In case of failure to comply with the deadlines, penalties shall apply. |
| 8. | Warranty period | A warranty shall be provided by the Supplier for the supplied vehicle, either under the manufacturer’s warranty or under the Supplier’s own responsibility, for a minimum mileage of 100,000 km or a period of at least 12 months from the date of commissioning. |
| 9. | Commissioning | All costs for additional work identified during the vehicle inspection shall be borne by the Supplier. |
| 10. | Quality evaluation criteria | - After commissioning, the vehicle and equipment shall be evaluated during the warranty period. Evaluation results shall comply with the manufacturer's specifications. - In the event of non-compliance with KGC requirements, the Supplier shall bear all costs for its elimination. |
| 11. | Confidentiality | Transaction data shall be confidential. The Contractor shall be liable for disclosure to third parties. |
| 12. | Environmental requirements | The vehicle shall comply with the environmental requirements established by the applicable laws of the Kyrgyz Republic. |
| 13 | Safety | The vehicle shall comply with the safety requirements established by the applicable laws of the Kyrgyz Republic. |

Appendix 1 to the Terms of Reference

| Terms of Reference for a NEFAZ-4208 shift bus on an all-wheel-drive KAMAZ-43114 chassis, or equivalent. |  |
| --- | --- |
| Purpose of the equipment | Intended for the prompt transportation of workers (repair and other crews) in the passenger compartment of a van-type body to working areas on public roads as well as off-road. |
| General characteristics |  |
| New vehicle (hereinafter - the vehicle), mileage not exceeding 1,000 km, year of manufacture: 2024-2025. | Yes |
| Engine |  |
| Type (diesel, petrol, gas, bifuel) | Diesel |
| Power, min. (hp) | 240 |
| Exhaust emission level (ecological class) | Not lower than EURO-2 |
| Air intake (with dryer filter, without dryer filter) | Multicyclone air cleaner with dust and water precleaning stage, without dryer. |
| Engine sump guard | Yes |
| Chassis |  |
| Wheel configuration | 6*6 |
| Front suspension | Spring-loaded |
| Rear suspension (pneumatic/balancing/spring, etc.) | Spring-balanced |
| Stabilizers on the front and rear axles | Yes |
| Front and rear fenders, mudguards, dirt protection covers on axles | Yes |
| Anti-roll barriers | No |
| Tires* | Tube-type tires with an off-road tread pattern. Size: 425/85R21 |
| Busbar arangement (single/double-slope) | Single |
| Transmission box |  |
| Type (Automatic/Mechanical) | Mechanical |
| Number of gears | 5/10 |
| Center and rear axles | With a centrally located two-stage main transmission |
| Rear axle differential lock | Yes |
| Transfer case |  |
| Type | Mechanical, two-stage with lockable center differential |
| Braking system |  |
| Type | Pneumatic, with drum-type brakes |
| Engine brake retarder | Yes |
| ABS anti-lock braking system | Yes |
| Electronic Braking System (EBS) | No |
| Parking brake, parking brake chamber lock valve | Yes |
| Exhaust system |  |
| Exhaust pipes must be directed to the side or, if structurally impossible, away from fuel lines and the fuel tank. | Exhaust pipe is standard, located on the right-hand side |
| Muffler heat shield | No |
| Fuel system |  |
| Fuel tank capacity | At least 300 liters |
| Lockable fuel tank cover | Yes |
| Fuel filter with water separator and heater | Yes |
| Fuel tank shall be located away from the battery or separated from it by an impermeable partition, and also positioned at a safe distance from the engine, electrical wiring, and exhaust pipe. Fuel tank shall be equipped with protective shields on the side walls and on the underside. | No |
| Electrical equipment |  |
| Electrical wiring shall be designed in accordance with the requirements of ADR 2015 and UNECE Regulation 105, Class FL. All wiring connections shall be made in junction boxes and properly insulated. | No |
| Battery pack | 2 pcs., 190 Ah each |
| Battery shall be located in a ventilated compartment made of metal or other material of equivalent strength with insulated internal walls. | Yes |
| Battery switch is a safety two-pole switch located in the cab and on the chassis, marked with an identifying symbol. The switch ensures that the contacts can be disconnected while the engine is running without causing dangerous overloads in the electrical circuit. | Yes |
| Generator: minimum 90 A. | Yes |
| One orange-colored flashing light installed on the roof. Mounting of the flashing light ensures secure installation under all vehicle operating conditions, and visibility of the light signal shall be provided at an angle of 360 degrees in the horizontal plane. | Yes |
| Headlamps (low and high beam) and fog lights | Yes |
| Reverse lights and fog lights | Yes |
| Front and rear position lights | Yes |
| Truck lights on the roof, with switch | No |
| Audible alarm (pneumatic/electric) | Electrical signal |
| Automatic beeping when driving in reverse | Yes |
| Interior cabin equipment |  |
| Interior cabin lighting, basic version | Yes |
| Three-point seat belts for the driver and passenger seats | Yes |
| Seat upholstery, textile | Yes |
| Pedals with rubber pads/anti-slip material | Yes |
| Power window door lifters. | No |
| Left-hand steering with hydraulic/electric booster | Yes, with power steering system |
| Adjustable steering wheel (reach and tilt) |  |
| Stand-alone heater (heater type: stand-alone cabin and engine heater/stand-alone cabin heater/engine block heater). | Engine pre-heater with the function of an independent passenger compartment heater, type 14TS-10 (equivalents considered). Additional cab heater of the PLANAR-4DM-24 type. |
| Conditioner | No |
| Instrument cluster in standard configuration with a tachometer and a speedometer. | Yes |
| Headlight level adjuster | Yes |
| Protective rubber floor mats | Yes |
| External cabin equipment |  |
| Cabin suspension (pneumatic/spring-loaded/semi-pneumatic/rigid). | Rigid suspension |
| Windshield - triplex, all other windows in the driver's cab made of tempered injury-safe glass. | Yes |
| Heated and electrically adjustable rearview mirrors (outside mirrors with normal and panoramic view on the right and left sides of the cab; mirror with curb view on the right side of the cab; mirror with view of the area in front of the front bumper). | Heated rearview mirrors, without electric drive |
| Headlight protection (steel mesh with a mesh size not exceeding 1 × 1 cm). | Yes |
| Insect (mosquito) screen in front of the radiator. | Yes |
| Vehicle is equipped with: |  |
| Tire inflation hose with a pressure gauge, with the possibility of connection to the vehicle’s brake system air reservoir. | Yes, length: at least 25 m. |
| Hydraulic jack | load capacity min. 20 tons |
| Safety stand for vehicles with a load capacity of not less than 12 tons. | No |
| At least two plastic anti-roll stops (for wheel diameter) | Yes |
| Two powder fire extinguishers, each with a capacity of at least 5 kg, mounted in the cabin. | Yes |
| Hammer for breaking glass in the cab and cutter for cutting seat belts. | Yes |
| First aid kit that meets the requirements of Kyrgyz legislation. | Yes |
| Two emergency stop signs | Yes |
| Set of hand tools | Recommended list (with spark-proof coating): cylinder wrench, screwdriver "+" and "-", pliers, tire mounting bar L-850 mm, D-22 mm, sledgehammer 3–5 kg, hammer |
| Set of spare lamps and fuses | Yes |
| Spare wheel with rim, same as mounted on the steering axle | Yes |
| Reflective safety vest | Yes |
| Name of the upper equipment installation: |  |
| Number of passenger seats (in the passenger compartment + in the cab): | 28+2 |
| Availability of seat belts: | According to the number of seats. |
| Bus seat requirements: | Two-seat, soft, injury-safe, with a non-adjustable/adjustable backrest. |
| Independent bus passenger compartment heater: | Yes |
| Possibility of emergency connection of the passenger compartment heater to the engine cooling system. | Yes |
| Cabin and passenger compartment radio system: | Radio receiver, intercom system |
| Compliance with the Technical Regulation on the Safety of Wheeled Vehicles. | Yes |
| Paintwork (the painting scheme is to be agreed with the Client in advance). | Yes |
| Documentation and other requirements |  |
| Operating manual and service book in Russian. | Yes |
| Driver training manual (main components and assemblies, operating rules). | Yes, included in the operating manual |
| Vehicle type approval with inclusion of confirmation of factory-built vehicle configuration in accordance with the requirements of ADR 2015 and UNECE Regulation 105 for a type FL vehicle. | No |
| Vehicle passport. | Yes |
| Passports and certificates for additional mounted equipment. | Yes |
| Design diagram of the vehicle indicating overall dimensions, weight parameters, and axle loads. | Yes |