Terms of Reference

For the purchase of equipment for automation of geomechanical monitoring of the in-pit dump #2.

| Item # | Requirements for the goods, works, services to be purchased (subject of purchase) | Purchase, delivery, installation, commissioning, and putting into operation of an automated geomechanical monitoring system (AGMS), including stationary chain-type automatic inclinometers and vibrating wire (string) piezometers. |
| --- | --- | --- |
| 1 | Name | Inclinometer boreholes: 6 units, depths: 74 m, 88 m, 95 m, 115 m, 105 m, 70 m. Piezometer boreholes: 6 units, with similar depths. Required components. |
| 2 | Place of goods delivery, provision of services/performance of work | Kumtor Mine site. Consider the harsh conditions of the highlands, climate and location of facilities. |
| 3 | Terms (periods) of goods delivery, provision of services, performance of work | The delivery period is 60 working days from the date of signing the service agreement. Installation and commissioning work shall be carried out in coordination with the Customer. |
| 4 | Order (sequence, stages) of work performance | Provide the work schedule to the Customer for approval. |
| 5 | Requirements for goods supplied, work performed, services rendered | - Automatic Inclinometers: Stationary automatic system (chain-type inclinometer). -Continuous automatic recording of displacements - Accuracy: no worse than ±0.1 mm/m. - Operating depth: 100 m and more. - Operating temperature range: from −40 °C to +50 °C. - Inclinometric guide casing: Inner diameter: 85 mm. -Material: Engineering Plastic (ABS or PVC) - Piezometers: Type: Vibration (VWP) or digital, suitable for automated data acquisition. Required characteristics: Accuracy: Not worse than ±0.1% of full scale - Data Acquisition and Transmission System: - Capability to connect automatic inclinometers and piezometers to a single cabinet of the Automated Monitoring System (AMS). - Measurement interval: from 10 minutes, with configurable settings. -Automatic sensor polling and data archiving. -Capability to transfer data to the Customer's server (GSM/LTE/WI-FI). -Local backup data storage. - Software: The software must be provided: - Generation of displacement profiles based on inclinometer data - Display of pore pressure variation graphs - Visualization of time series data - Configuration of threshold values and alarm notifications -Data export in CSV/XLSX formats - User access level management. - Installation and commissioning: -Borehole drilling is performed by the Customer (Ø153mm) - Installation of chain-type inclinometers and piezometers in the design boreholes - Ensuring proper fixation of the measuring systems along the entire borehole depth - Sealing of borehole collars - Connection of sensors to the data acquisition system - Calibration and functional testing of the equipment - Conducting trial (pilot) operation. - Supplier Requirement: - Proven experience in performing similar projects (with specification of project locations and the possibility of requesting completed works for verification) - Availability of qualified engineering personnel. |
| 6 | Procedure for Delivery and Acceptance of Goods, Services, Work Results | The Supplier must ensure: -Delivery of the equipment as a complete set -Provision of operating manuals and instructions in Russian -Software installation and personnel training - A warranty period of not less than 12 months -Technical support, both online and offline, upon request - Performance of equipment verification/calibration upon request -Minimum possible delivery time. |
| 7 | Requirements for the transfer of technical and other documents to the Customer upon completion and acceptance of the works | The Supplier shall provide the Customer with the manufacturer’s technical documentation, certificates, technical data sheets, and documents confirming the quality of materials, products, and equipment used in the works. All supplied equipment shall be new, unused, and shall comply with the requirements of this Terms of Reference (ToR). |
| 8 | Warranty obligations | If a defect or non-compliance with the specified parameters of the supplied goods is identified, the Supplier shall be obliged to replace the goods or refund the purchase price. |