TERMS OF REFERENCE

for the purchase of a mobile oil unit for Kumtor Mine,

Kumtor Gold Company CJSC

| Item № | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1. | Delivery location | 9 Naryn highway, Balykchy, Kyrgyz Republic. |
| 2. | Client | Kumtor Gold Company CJSC. |
| 3. | Purpose of the purchase | The subject of these Terms of Reference is the purchase two mobile oil units mounted on a SHACMAN X3000 6×6 chassis or equivalent. |
| 4. | General requirements / Regulations and standards | - The supplied unit (hereinafter referred to as the equipment) shall be new and unused. Type: a truck equipped with an oil unit classified as an N3 category vehicle. Purpose: operation on public roads for fuel and lubricants transportation and refueling. The equipment will be operated on paved, unpaved, and mountain roads in high altitude conditions above 4,000 m above sea level, with the temperature range from - 35℃ to +40℃. - The equipment shall meet or exceed the requirements of the technical specifications in terms of performance and energy efficiency. - All necessary user manuals and technical documentation shall be included. - The equipment shall comply with the applicable safety requirements established by law. - When supplying the equipment, the Supplier shall provide originals of certificates of conformity and declarations of conformity to the requirements of regulatory documents for the supplied goods in the territory of the Kyrgyz Republic/EAEU. - The equipment supplied shall not be pledged, under arrest or another encumbrance. |
| 5. | Technical specifications | 1. General characteristics. 1.1 Year of Manufacture: 2025 or later. 1.2 Mileage: no more than 1,000 km. 2. Engine. 2.1 Type: diesel. 2.2 Power: 420 hp and above. 2.3 Environmental class: EURO-3. 3. Chassis. 3.1 Wheel configuration: 6×6. 3.2 Front suspension: spring-loaded. 3.3 Rear suspension: spring balancing. 3.4 Mud protection: front and rear fenders, mud flaps, and axle-mounted mud guards. 3.5 Anti-rollback barriers: side barriers. 3.6 Tires: to be considered according to the chassis. 3.7 Tire arrangement: dual tires. 4. Transmission. 4.1 Type: mechanical with retarder. 4.2 Axle differential lock. 5. Braking system 5.1 Type: pneumatic, with drum type brakes. 5.2 Engine brake retarder. 5.3 Parking brake, parking brake chamber lock valve. 6. Exhaust system. 6.1 Exhaust pipes shall be positioned outside the fuel lines and fuel tank area. 6.2 Muffler heat shield: muffler enclosure. 6.3 Spark arrester. 7. Fuel system. 7.1 Fuel tank capacity: 300 liters or more. 7.2 Lockable fuel tank cover. 7.3 Fuel filter with water separator. 7.4 Fuel tank shall be located away from the battery or separated from it by an impermeable partition and positioned at a safe distance from the engine, electrical wiring, and exhaust pipe. Fuel tanks shall be equipped with protective shields on the side walls and on the underside. 8. Electrical equipment. 8.1 Electrical system voltage: 24 V. 8.2 Battery: 2 units, 190 Ah each. 8.3 Battery shall be installed in a ventilated compartment made of metal or another material of equivalent strength, with insulated internal walls. 8.4 Battery switch is a safety two-pole switch located in the cab and on the chassis, marked with an identifying symbol. The switch ensures that the contacts can be disconnected while the engine is running without causing dangerous overloads in the electrical circuit. 8.5 Generator: minimum 90 A. 8.6 Pneumatic/electric sound signal. 8.7 Automatic reverse warning signal. 9. Vehicle configuration. 9.1 Hydraulic jack with a lifting capacity of at least 20 tons. 9.2 Tire inflation hose, minimum 20 m. 9.3 At least two-wheel chocks (sized for the wheel diameter). 9.4 Two dry powder fire extinguishers, each with a capacity of at least 5 kg, mounted in the cabin and on the unit. 9.5 First aid kit that meets the requirements of Kyrgyz legislation. 9.6 Two emergency stop warning signs. 9.7 Spare wheel with rim, identical to those installed on the steerable axle. 10. Equipment for the oil station van. 10.1 The van shall be insulated. 10.2 Equipped with a compressor driven by a PTO (Power Take-Off) for operating the oil transfer/pumping equipment. Type: Piston or screw type. Capacity: 1,600 L/min or higher. Operating pressure: 8-10 bars. Receiver volume: 500-600 liters. 10.3 Equipped with a heating system. 10.4 Availability of lighting. 10.5 Rear swing doors and additional side service/access doors. 11. Documentation 11.1 Spare parts catalog indicating part numbers, provided in both printed (hard copy) and electronic (PDF) formats. 11.2 Vehicle passport. 12. Additional information 12.1 Adaptation for operation in high-altitude and cold-climate conditions. 12.2 Technical specifications of the oil dispensing unit are provided in Appendix 1. |
| 6. | Payment and delivery terms | - Payment terms: to be agreed with the Client. - The vehicle shall be delivered by manufacturer to BMY warehouse in Balykchy within the deadlines specified by the Client. - Assembly and commissioning shall be completed within the deadlines specified by the Client. - In case of failure to comply with the deadlines, penalties shall apply. |
| 7. | Acceptance | - The vehicle shall be accepted at the Client's premises in the presence of the Supplier. - All costs for additional work identified during the vehicle inspection shall be borne by the Supplier. |
| 8. | Warranty period | - The Supplier shall provide a warranty period for the purchased equipment in accordance with the manufacturer's terms and conditions. |
| 9. | Training and operation manual | - The vehicle's operation manual shall be provided as part of the package. - Training shall be provided to personnel on the operation of the oil dispensing unit. |
| 10. | Quality evaluation criteria | - After commissioning, the vehicle shall be evaluated during the warranty period. Evaluation results shall comply with the manufacturer's specifications. - In the event of non-compliance with KGC requirements, the Supplier shall bear all costs for its elimination. - The vehicle shall comply with the safety requirements established by the applicable laws of the Kyrgyz Republic/EAEU. |
| 11. | Documentation requirement | - Complete set of documentation required for the vehicle registration with the State Agency for Registration of Vehicles and Drivers (UNAA) in accordance with the KR Government Decree. Copies of cargo customs declarations. - Warranty and service book, operation manual in Russian, in both paper and electronic formats, shall be available. Spare parts catalog shall be included. |
| 12. | Confidentiality | - Transaction data shall be confidential. Disclosure to third parties shall be prohibited. |

Appendix 1

Technical requirements for the oil dispensing unit

1. General characteristics

- Compressed air operation.

- Support for pneumatic pumps for the supply of oils and lubricants.

- Possibility to connect to a compressor with adjustable pressure.

- Availability of dispensing guns with a dosage function.

- Automatic hose reels (as required).

- High flow rate of about 100 l/min.

- Filling into tanks via externally accessible valves.

- Equipped with independent heaters.

2. Tanks and volumes

- Tanks equipped with level gauges.

- Total storage capacity of oils and liquids: 7,700 liters.

| Type of liquid | Grade/viscosity | Minimum volume |
| --- | --- | --- |
| Engine oil | 15W40 | 1,000 l |
| Cooling liquid | Antifreeze | 650+ liters |
| Hydraulic oil | 10W | 1,500 l |
| Hydraulic oil | Hitachi 46 | 1,500 l |
| Transmission oil | HD-50 | 1,000 l |
| Transmission oil | 75W90 | 650+ liters |
| Compression oil | ISO-30 | 400+ liters |
| Used oil |  | 1,500 |

3. Lubricants (solidol)

Storage and filing support:

Solidol Black (black): 1 barrel.

Solidol Ulti-plex (white): 2 barrels.

4. Additional requirements

- Integration with inventory management system (e.g., Graco Matrix).

- Resistance to industrial conditions:

- Dust

- Vibrations

- Temperature changes

- Easy maintenance and replacement of components.

Equipment layout drawing

Layout of dispensing guns and reels

Reel 10 is combined with Reel 3 connected to the Hitachi reservoir.

Reel 11 is combined with Reel 2 connected to the 10W hydraulic oil reservoir.

| 1 | Antifreeze |
| --- | --- |
| 2 | 10W hydraulic oil reservoir |
| 3 | Hitachi hydraulic oil reservoir |
| 4 | HD/50 |
| 5 | 75/90 |
| 6 | Compressor oil |
| 7 | Lubricant |
| 8 | Engine oil |
| 9 | Air |
| 10 | Hitachi hydraulic oil reservoir |
| 11 | 10W hydraulic oil reservoir |
| 12 | Used oil |