TERMS OF REFERENCE

for the purchase of Infrared Heaters

| Item № | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1 | Location | Issyk-Kul Region, Jeti-Oguz District, Kumtor Gold Mine. |
| 2 | Client | Kumtor Gold Company CJSC. |
| 3 | Purpose of the purchase | In order to ensure the efficient use of electrical energy and improve the operational safety of heating systems, it is planned to implement a pilot project to replace the existing energy-intensive air heaters with infrared heaters in warehouse facilities and garages, with the purpose of evaluating their energy efficiency, operational reliability, and feasibility for further use under industrial conditions. |
| 4 | Name of the equipment | Name of the equipment: Infrared Heater. Model: UFO TYY 90. Type: Industrial Electric Infrared Heater. Purpose: Heating of industrial, semi-open and open areas, as well as industrial premises. |
| 5 | Technical specifications | Rated (installed) power:; 9,000 W.. Supply voltage:; 380 V.. Supply frequency:; 50 Hz.. Type of heating:; Infrared.. Recommended heating area (indoor):; up to 90 m2.. Recommended heating area (outdoor):; up to 60 m2.. Overall dimensions (H×W×D):; about 480 ×1,000 × 130 mm.. Installation method:; wall/ceiling mounted.. Operating mode:; continuous operation.. |
| 6 | Design and operational requirements | - The housing shall be made of materials resistant to industrial operating conditions. - The Heater shall provide directional infrared heating without heating the air throughout the entire volume of the premises. - The design shall allow fixed installation on a wall or ceiling. - The equipment shall be designed for continuous operation in industrial conditions. |
| 7 | Delivery completeness | - Infrared Heater: 10 pcs. - Mounting kit for wall/ceiling installation. - Product passport. - Operating manual. |
| 8 | Requirements for the supplier | - The equipment shall be new and not used. - The equipment shall be delivered in factory packaging ensuring safety and protection during transportation. - Warranty: at least 24 months from the date of commissioning. - Delivery period: 30 calendar days. |
| 9 | Acceptable equivalents | Supply of equivalent equipment is permitted only subject to full compliance with the technical and operational characteristics specified in these Terms of Reference. |