Terms of Reference

for the purchase of a paint coating thickness gauge for Component Rebuild Workshop of Maintenance Department, Kumtor Gold Company CJSC

| Item № | Required parameters for the Goods to be purchased | Specific requirements for the Goods |
| --- | --- | --- |
| 1 | Description and quantity | Paint coating thickness gauge for Component Rebuild Workshop of Maintenance Department. Stock Number: 01.02.207. |
| 2 | Delivery location | According to the Agreement terms. |
| 3 | Client | Kumtor Gold Company CJSC. |
| 4 | Contractor | To be selected according to the Bid results. |
| 5 | Delivery deadline | To be specified in the Agreement. The shortest delivery period will be considered an advantage. |
| 6 | Requirements for the Goods to be supplied | - The proposed Goods shall be new, unused, certified or declared, fully compliant with the applicable standards of the Kyrgyz Republic/EAEU. - The proposed Goods shall be produced in compliance with the requirements of regulatory documents (standards, technical specifications, quality certificates approved in accordance with the established procedure for this type of Goods, etc.) under the serial production conditions. - The proposed Goods shall comply with the functional, technical and quality performance characteristics (operational characteristics) of the purchase object specified in this section. 4. The Goods shall comply with the applicable safety requirements established by law. 5. Technical Specifications: - Measurement principle: combined (electromagnetic and eddy-current sensor). - Sensor: ruby. - Substrate detection: Fe–NFe (ferrous metal and aluminum). - Dimensions: 100x62x25 mm (± 5 mm). - Measurement range: 0–3,500 µm. - Error: ±1-3%. - Response time: up to 0.5 s. - Measurement resolution: 1–2 µm. - Measuring probe type: built-in. - Auto calibration: required. - Galvanized coating detection: yes. - Screen: LED or OLED. - Operating temperature: -35 0 C to +40 0 C. - Weight: not more than 150 g. - Protection rating: IP40 or higher. - Power supply: AAA batteries or rechargeable battery (Type-C). - Purpose: intended for checking the thickness of paint, lacquer, and body filler layers on vehicle bodies after bodywork repairs in the Component Rebuild Workshop. |
| 7 | Goods delivery and acceptance procedure | 1. When supplying the paint coating thickness gauge, the Supplier shall provide: originals or duly certified copies of valid certificates of conformity and declarations of conformity to the requirements of regulatory documents for the supplied Goods in the territory of the Kyrgyz Republic/EAEU. 2. In the event of non-compliance with KGC requirements, the Supplier shall bear all costs for its elimination. |
| 8 | Requirements for the submission of technical and other documents to the Client. | - The Supplier shall, upon delivery of the Goods, submit to the Client the consignment notes, invoices, as well as other required documents, including documents confirming the Manufacturer's/Supplier's quality warranty and the operating instructions (manual). - All required user manuals and technical documentation shall be included in the package and shall not be accepted in the form of copies. |
| 9 | Warranty obligations | 1. The Supplier shall guarantee the quality of the Goods according to the Manufacturer’s terms. |

The photographs below are provided as examples of the required tool.