Terms of Reference

for the purchase of a grinding machine for Component Rebuild Workshop of Maintenance Department, Kumtor Gold Company CJSC

| Item № | Required parameters for the Goods to be purchased | Specific requirements for the Goods |
| --- | --- | --- |
| 1 | Description and quantity | Grinding machine for Component Rebuild Workshop of Maintenance Department. Stock Number: 01.02.200. |
| 2 | Delivery location | According to the Agreement terms. |
| 3 | Client | Kumtor Gold Company CJSC. |
| 4 | Contractor | To be selected according to the Bid results. |
| 5 | Delivery deadline | To be specified in the Agreement. The shortest delivery period will be considered an advantage. |
| 6 | Requirements for the Goods to be supplied | - The proposed Goods shall be new, unused, certified or declared, fully compliant with the applicable standards of the Kyrgyz Republic/EAEU. - The proposed Goods shall be produced in compliance with the requirements of regulatory documents (standards, technical specifications, quality certificates approved in accordance with the established procedure for this type of Goods, etc.) under the serial production conditions. - The proposed Goods shall comply with the functional, technical and quality performance characteristics (operational characteristics) of the purchase object specified in this section. 4. The Goods shall comply with the applicable safety requirements established by law. 5. Technical Specifications: - Type: eccentric. - Body: plastic. - Speed: 4,000–10,000 rpm (adjustable). - Motor type: brushless. - Power: 350-400 Watts. - Power supply: mains powered. - Mains voltage: 220–240 V. - Cable length: 4-7 m. - Vacuum cleaner connection for dust extraction: yes. - Plate diameter: 150 mm. - Abrasive attachment type: hook-and-loop (Velcro). - Weight: up to 1 kg. - Vibration level: 3.3–3.5 m/s2. - Noise level: up to 70 dB. - Grinding stroke: 5-6 mm. - Package contents: storage case. - Purpose: intended for leveling, cleaning, and polishing vehicle body surfaces during body repair operations in the Component Rebuild Workshop. |
| 7 | Goods delivery and acceptance procedure | 1. When supplying the grinding machine, the Supplier shall provide: originals or duly certified copies of valid certificates of conformity and declarations of conformity to the requirements of regulatory documents for the supplied Goods in the territory of the Kyrgyz Republic/EAEU. 2. In the event of non-compliance with KGC requirements, the Supplier shall bear all costs for its elimination. |
| 8 | Requirements for the submission of technical and other documents to the Client. | - The Supplier shall, upon delivery of the Goods, submit to the Client the consignment notes, invoices, as well as other required documents, including documents confirming the Manufacturer's/Supplier's quality warranty and the operating instructions (manual). - All required user manuals and technical documentation shall be included in the package and shall not be accepted in the form of copies. |
| 9 | Warranty obligations | 1. The Supplier shall guarantee the quality of the Goods according to the Manufacturer’s terms. |

The photographs below are provided as examples of the required tool.