Terms of Reference

Foam Generator GVPP-200

for Kumtor Underground Mine

| Item № | List of basic data and requirements | Basic data and requirements |
| --- | --- | --- |
| 1 | Delivery location | KUMTOR GOLD COMPANY CJSC, 9 Naryn highway, Balykchy, Kyrgyz Republic. |
| 2 | Client | Kumtor Gold Company CJSC, Underground Mining Department. |
| 3 | General provisions | Operating Principle: the generator operates by supplying a pressurized flow of working liquid to the nozzle. Due to the ejector effect, as the atomized jet enters the manifold, air is entrained and mixed with the solution. As the resulting air-liquid mixture passes through the mesh, foam is generated. |
| 4 | Technical specifications | Technical specifications: - Material: thin-gauge steel sheet. - Technical Specifications. Parameter values (nominal): Foam output capacity: 200 L/s. Consumption of 4–6% PO-1 type foam concentrate solution: 1.6–2.0 L/s. Pressure upstream of the nozzle: 0.4–0.6 MPa (4–6 kgf/cm²). Foam expansion ratio: 100 ± 30. Foam delivery range: minimum 10 m. Overall dimensions: 430 × 220 mm. Weight: maximum 2.5 kg. Note: the foam output capacity is determined at the maximum values of foam concentrate solution flow rate and foam expansion ratio. |
| 5 | Requirements for components and consumable materials | Originality and quality - All components and consumable materials shall be new, genuine, and manufactured by the original equipment manufacturer (OEM). - Equivalent products used or refurbished components shall not be accepted. |
| 6 | Quality and certification | Compliance with standards and requirements - All supplied products shall comply with the manufacturer's requirements and applicable quality standards (GOST, TU, ISO, where applicable). Quality Certificate - The Supplier shall provide a Certificate of Quality issued by the manufacturer for each batch. - The certificate shall include: • Date of manufacture. • Confirmation of compliance with the technical requirements. Certificate of conformity/declaration - If the product is subject to mandatory certification, the Supplier shall provide a Certificate of Conformity or Declaration of Conformity in accordance with applicable legislation. Product passport/technical documentation - Where applicable (for technical products), the following shall be provided: • Product passport. • Technical description. • Specification. • Information on service life and operating conditions. Instructions and description - For products requiring installation, commissioning, adjustment, or operation, the Supplier shall provide operating instructions in Russian (or in the manufacturer's language, where permitted). Quality requirements - The products shall be free from defects, damage, corrosion, and any other signs of compromised integrity. - The packaging shall be of sufficient quality to ensure the integrity and protection of the product during transportation and storage. |
| 7 | Service life and warranty | 1. Product service life - The service life shall comply with the manufacturer's specifications and shall be no less than the service life specified in the technical documentation. - The products shall be supplied with the current date of manufacture (not more than 12 months from the date of delivery). - The materials shall have a shelf life sufficient to allow their use throughout the entire stated service life. 2. Warranty obligations - The Supplier shall provide a warranty of at least 12 months from the date of delivery, or in accordance with the manufacturer's warranty (if longer). - The warranty shall cover manufacturing defects, non-conformities, and latent defects. - In the event of a defective product, the Supplier shall, at its own expense: • Replace the defective product. • Repair the product, where applicable. • Provide a refund (subject to mutual agreement). The warranty shall include the Supplier's commitment to maintain the availability of spare parts throughout the entire service life of the product. |
| 8 | Packaging and marking | 1. Product packaging Each product shall be individually packaged in a sealed, dust- and moisture-resistant bag. The outer packaging shall consist of a sturdy cardboard box with cushioning materials (foam and bubble wrap). The packaging shall prevent mechanical damage and deformation of the product. 2. Packaging marking Product name. Catalog number. Quantity. Manufacturer. Handling symbols/markings: “Fragile,” “Handle with Care,” “Keep Dry”. |
| 9 | Acceptance conditions and selection criteria | The product shall be accepted based on: - Visual inspection. - Verification of product and packaging markings. - Documentation review: availability of a complete set of documentation in Russian, including pressure-to-force conversion tables. |
| 10 | Payment and delivery terms | - Payment terms: as specified in the Agreement. - The product shall be delivered at the Supplier’s expense and responsibility. - Delivery terms: for non-residents of the Kyrgyz Republic – DAP, for residents of the Kyrgyz Republic – DDP. - The product shall be delivered in accordance with the approved schedule. - In the event of a delay in delivery, the Supplier shall be liable in accordance with the Agreement terms. |